# High-level summary

LTI systems:

- Described entirely by impulse response h. Any output y can be calculated as the convolution of the input x with the impulse response h.

- The step response is the integral or accumulation of the impulse response. The impulse response is the derivative or differencing of the step response.

- The steady-state response (system output) to a sinusoid at frequency f0 is a sinusoid at f0 multiplied by the frequency response of the system Hf0 (for DT LTI, it’s w0 and Hejw0).

LCCDE:

- Special class of LTI systems. Important because LCC differential equations represent real circuits, while LCC difference equations are necessary to implement IIR.

- Characterized entirely by their constant coefficients. By convention, bk are the coefficients for the input terms, and ak are the coefficients for the output terms. The highest order of ak is the order of the system.

Frequency domain/CT Fourier series:

- Each frequency component is characterized by frequency, amplitude, and phase.

- A periodic signal with fundamental frequency f0 and period T0 can be decomposed into orthogonal frequency components. The frequencies are harmonics of f0, and the amplitude and phase of each component is the Fourier series coefficient, Xk, where k denotes the harmonic order.

- A delay, xt-t0, causes a phase shift at each frequency component of e-j2πkf0t0.

- For a real signal, x*t=xt, X-k=Xk*. That is, Xk are conjugate symmetric.

- For a real even signal, x-t=xt, Xk are real.

- For a real odd signal, x-t=-xt, Xk are imaginary.

- For a real signal with odd half-wave symmetry, xt±T02=-xt, even frequency components are null.

- The power of a periodic signal is equal to kXk2.

- For a pulse train, Xk follow a sinc function, sinckf0τ. Sidelobe width is given by kfo=nτ or k=nToτ.

  - If you keep τ constant, the nulls are always at ±1τ, ±2τ, etc. regardless of To.

  - If you keep To constant, decreasing τ pushes out the nulls.

  - For τ=T02, we have a square wave, and even harmonics are 0. Odd harmonics roll off as 1k.

- Due to discontinuous jump in pulse train, synthesizing pulse train from Fourier series does not converge point-by-point. Gibbs phenomenon/ringing.

- For a triangle wave, even harmonics are null, and odd harmonics roll off as 1k2.

CTFT:

- Formally, CTFT only exists for aperiodic energy signals, but with FT in the limit, it can be applied to periodic power signals.

- CTFS is a sampled version of CTFT when you analyze one period of a periodic signal.

- Xf is the spectrum/energy spectral density of xt. If xt has units of V, then Xf has units of VHz. Integrating over xt2 or Xf2 yields the total energy in the signal.

- Ideal filters are not realizable. Realizable filters include Butterworth, Chebyshev, Bessel, elliptic, and are defined by LCC differential equations.

- Same symmetry properties as CTFS

Sampling:

- CT-DT: wo=2πfofs, fs↔2π

- Sampling = convolving with freq domain impulse train at kfs. When converting to DT, DT spectrum is 2π periodic, which means feq=fo±kfs. Signals at fo±kfs appear at same DT frequency wo (aliasing), and signal at fo appears at wo±2πk.

  - CT freq domain: Xsf=Xf*Sf=fsmXf-mfs. Xf-mfs are called the spectral translates.

- Nyquist-Shannon sampling theorem: fs>2fo, 2fo is the Nyquist rate and fo is the Nyquist frequency.

- Principle alias lies in -fs2,+fs2. This is the alias that is reconstructed. fprinciple=fo-kfs∈-fs2,+fs2, fo∈kfs±fs2.

- Quantization (B is number of bits)

  - Δ=2Vm2B, levels = -Vm to +Vm-Δ

  - Quantization noise power = e2=Δ212

  - SQNR for full wave sinusoid = 6.02B+1.76dB

- Reconstruction: return to sampled CT signal (signal + images) and then filter the images

  - Xsf=mXf-mfs

  - ZOH (realizable DAC): ht=Πt-Ts2TsFHf=Tssincffse-jπfTs. Nulls at kfs provide partial image suppression.

  - Ideal reconstruction LPF is a time-domain sinc: Hf=Πf2WFht=2Wsinc2Wt, xt=xst*ht

  - To relax the design of the analog interpolation filter, increase DAC sampling rate to push out the images.

Fourier transforms:

- Energy signals

  - If a signal is in L1 space, then its FT exists and converges pointwise (as the limits of integration or summation go to ±∞).

    - L1 space means x1<∞. x is absolutely integrable or absolutely summable.

    - CT: x1=-∞∞xtdt

    - DT: x1=n=-∞∞xn

  - If a signal is in L2 space, then it is an energy signal, and its FT exists and converges in the mean-square sense. It’s not guaranteed to converge pointwise.

    - L2 space means x2<∞. x is square integrable or square summable.

    - CT: x2=-∞∞xt2dt

    - DT: x2=n=-∞∞xn2

  - X2 is the energy spectral density and has units of JHz or V2sHz

- Power signals: you must use the technique of FT in the limit to calculate the FT

  - X2 is the power spectral density and has units of WHz or V2Hz

  - PSD is ESD taken to the limit (I think)

- Convergence: uniform vs. pointwise vs. absolutely vs. mean-square. Author calls it uniform convergence for LT.

- Autocorrelation: ESD/PSD is FT of autocorrelation function

- Symmetry properties for real signals

  - x real  X is conjugate symmetric, X and ReX are even, ∠X and ImX are odd

  - x real and even  X is real and even

  - x real and odd  X is imaginary and odd

Program examples of each convergence type (L1, L2, power)

You can use scipy to plot the FT of a signal that is real and even because FFT treats the signal as infinite duration, so you can effectively get a “non-causal” signal.

Fourier transform theorems and pairs:

- Linearity

- Time shift

- Frequency shift

- Parseval’s/Plancherel’s – is this assuming energy signal?

- Downsample

- Upsample

CT-DT relationship:

- Xsf=1Tsm=-∞∞Xf-mfs=Xejw with the change of variable w=2πffs

Fourier transform convergence (convergence meaning as the time or frequency limits go to infinity):

- L1: the signal is absolutely integrable (CT) or absolutely summable (DT). The FT exists and converges pointwise. Can also say that the signal is part of L1 space.

- L2: the signal is an energy signal – square integrable (CT) or square summable (DT). The FT exists and converges in the mean-square sense (mean-square error goes to zero) but not pointwise. Can also say that the signal is part of L2 space.

- Power signals: use FT in the limit to get the FT, which will be impulses in the frequency domain. (Not necessarily – more generally it defines a PSD, which is continuous for stationary processes)

- Convergence goes the other way too. For example, if you start with a time-domain sinc, then you get the Gibbs phenomenon in frequency domain because of mean-square convergence. If you start with a freq-domain sinc, then you get the Gibbs phenomenon in time-domain. Another example of duality of FT.

Energy signals have ESD (energy spectral density) while power signals have PSD. These have different units. To compare ESD to PSD, you have to either limit the time window of the power signal, or you have to repeat the energy signal into infinity.

Autocorrelation

DFT: transform domain signal processing: in practice, at least for the digital front-end, we don’t do this. It’s still an approximation of convolution, which is easily realized with digital hardware. But I suppose in the baseband side, we have access to FFTs, which allows us to do freq-domain signal processing. Very useful for OFDM. FFT is basically like a filter. Xk is the output signal, and each Xk is created by multiplying the input signal with a bunch of coefficients (defined by WNkn) and summing.

Convolution with impulse/sifting

Convergence requirements, FT in the limit

I guess I never really wrapped my head around the fact that energy signals are time-limited and that CTFT/DTFT represents the energy because the frequency domain doesn’t have any notion of time. Somehow all of these complex exponentials add up together to produce a signal that is time-limited.

Have a good picture of CT-DT relationship

- CTFS: continuous-time periodic signals, discrete spectrum, power signals

- CTFT: continuous-time signals, continuous spectrum, energy signals, energy spectral density

- DTFT: discrete-time signals, continuous spectrum, energy signals, energy spectral density

- DFT: discrete-time signals, discrete spectrum

Stability, poles, time-domain signal/impulse response

Complex conjugate poles – residues are also conjugates. Results in real signal. How does this relate to conjugate symmetry? This must be true for both DT and CT.

Complex conjugate poles and real poles only – this means FT is conjugate symmetric, which means the time-domain signal or impulse response is real. That also means that if you use PFE to get the residues, the residues of conjugate poles must be conjugates. This also means that if you multiply out the s-pk terms to get the denominator polynomial, all coefficients will be real.

This is true for both DT (z domain) and CT (s domain).

# Topics

Reference: Signals and Systems for Dummies by Mark Wickert

# Math review

## Partial fraction expansion (PFE)

Important for inverse Laplace and z-transforms. PFE allows you to split up a ratio of polynomials into a sum of fractions, with each denominator containing a single or repeated root.

Consider Nth and Mth-order polynomials

Ds=aNsN+aN-1sN-1+…+a1s+a0

=s-p1s-p2⋯s-pN-1s-pN

Ns=bMsM+bM-1sM-1+…+b0

For M<N, NsDs is a proper rational function. If you assume Ds has no repeated roots, then the PFE of NsDs is

NsDs=A1s-p1+A2s-p2+…+ANs-pN

If you don’t assume no repeated roots, and each of r roots has multiplicity Ni such that N1+N2+…+Nr=N, then

NsDs=k=1N1A1ks-p1k+k=1N2A2ks-p2k+…+k=1NrArks-prk

When M≥N (improper rational function), you need to use long division to reduce the numerator order to be less than the denominator order. You need to first put your ratio of polynomials in the form

NsDs=Ks+RsDs

Where RsDs is a proper rational function.

For distinct (no repeats) roots, the coefficients are given by

Ai=NsDss-pis=pi

For repeated roots, the coefficients are given by

Aik=1Ni-k!dNi-kdsNi-ks-piNiNsDss=pi

## Trigonometry

Useful trig identities:

- sinu=cosu-π2

- cosu=sinu+π2

- cosu±2πk=cosu

- cos-u=cosu

- sin-u=-sinu

- cos2u-sin2u=cos2u

- 2sinucosu=sin2u

- cos2u=121+cos2u

- sin2u=121-cos2u

- sinu±v=sinucosv±cosusinv

- cosu±v=cosucosv∓sinusinv

- cosucosv=12cosu-v+cosu+v

- sinusinv=12cosu-v+cosu+v

- sinucosv=12sinu-v+sinu+v

## Sinc function

sincx=sinπxπx

Nulls at x is non-zero integer.

TD sinc = ideal LPF

TD sinc used for ideal pulse shaping

FD sinc used for OFDM

## Exponential and logarithmic functions

The logarithm of a number, x, is the exponent by which another fixed value, the base b, must be raised to produce that number.

x=by

y=logbx

Ex. 1000=103↔3=log101000.

Exponentiation and logarithm are inverses, that is,

logbby=y

blogbx=x

Exponentiation turns sums into products, while logarithms turn products into sums.

bn+m=bnbm

lognm=logn+logm

Common bases:

- Base 10, aka decimal or common logarithm

- Base e≈2.71828, aka natural logarithm, which corresponds to the exponential function

- Base 2, aka binary algorithm

Logarithmic scales reduce wide-ranging quantities to smaller scopes (like decibels).

To change from base k to base b,

logbx=logkxlogkb

Identities:

- logbb=1

- logb1=0

- b0=1

- b1=b

- logbxy=logbx+logby

- logbyx=logby-logbx

- logbxp=plogbx

- logbpx=logbxp

Derivatives:

- ddxex=ex

- ddxbx=bxlnb

- ddxlnx=1x

- ddxlogbx=1xlnb

## Complex arithmetic

Rectangular form (x-y coordinates): z=x+jy

Polar form (magnitude/envelope and phase): z=rejϕ

Rectangular to polar conversion:

r=z=x2+y2

ϕ=arctany/x

Polar to rectangular conversion:

x=rcosϕ

y=rsinϕ

Let zi=xi+jyi=riejϕi.

z1+z2=x1+x2+jy1+y2

z1-z2=x1-x2+jy1-y2

z1z2=r1r2ejϕ1+ϕ2=x1x2-y1y2+jx1y2+y1x2

z1z2=r1r2ejϕ1-ϕ2=x1x2+y1y2-jx1y2-y1x2x22+y22

conjz≡z*=x-jy=re-jϕ

zz*=r2=z2

z+z*=2⋅Rez

z-z*=2j⋅Imz

Euler’s identity:

ejϕ=cosϕ+jsinϕ

rejϕ=rcosϕ+jrsinϕ

cosϕ=ejϕ+e-jϕ2

sinϕ=ejϕ-e-jϕ2j

Example:

1+e-jw0t=e-jw0t2ejw0t2+e-jw0t2=2e-jw0t2cosw0t2

### Phasor analysis

Phasor analysis: when sinusoidal signals of the same frequency w0 are added together (superimposed), the result is a single sinusoidal signal having a composite amplitude and phase. Mathematically,

k=1NAkcosw0t+ϕk=Acosw0t+ϕ=A⋅Reejw0t+ϕ

Aejϕ=k=1NAkejϕk

The composite amplitude and phase are given by the vector addition of the individual amplitudes and phases.

Proof:

k=1NAkcosw0t+ϕk=k=1NAkReejw0t+ϕk=Rek=1NAkejw0t+ϕk=Reejw0tk=1NAkejϕk=Reejw0tAejϕ=ReAejw0t+ϕ=Acosw0t+ϕ

## Calculus

Formally, fx has derivative f'x at x if this limit exists:

dfxdx=f'x=limΔx→0fx+Δx-fxΔx

Differentiation formulas (a is constant and u and v are functions of x):

- ddxaxn=naxn-1

- ddxuv=udvdx+vdudx

- ddxuv=vdudx-udvdxv2

- dyudx=dydududx (chain rule)

- ddxcosu=-sinududx

- ddxsinu=cosududx

- ddxarctanu=11+u2dudx, -π2<arctanu<π2

- ddxlnu=1ududx

- ddxecx=cecx

- ddxeu=eududx

Indefinite integrals are also known as antiderivatives, i.e. if

Fx=fxdx

Then

fx=ddxFx

Useful indefinite integrals:

- sinaxdx=-1acosax

- cosaxdx=1asinax

- eaxdx=eaxa

- xeaxdx=eaxa2ax-1

Useful definite integrals:

- 0∞sinaxxdx=π2,a>0

- 0∞xne-axdx=n!an+1,n integer, a>0

- 0∞e-axcosbxdx=aa2+b2,a>0

- 0∞e-axsinbxdx=ba2+b2,a>0

When a closed-form solution for integration is not available, use numerical integration (e.g. SciPy integrate module, quad function).

To find the minimum or maximum of a function fx, find

f'x0=0

(As long as there is only one global minimum or maximum).

- If minimum, f''x0>0

- If maximum, f''x0<0

Integration by parts:

abuxv'xdx=uxvxab-abu'xvxdx

v'xdx=dvdxdx=dv

u'xdx=dudxdx=du

Compact form of integration by parts:

u⋅dv=uv-v⋅du

### Geometric series

Geometric series are often found in discrete-time signals and systems.

A finite geometric series, aka the sum of a finite initial segment of an infinite geometric series:

Sn=a+ar+ar2+ar3+…+arn=k=0nark=a1-rn+11-r

The convergence of the infinite sequence of partial sums of the infinite geometric series depends on r, where r may be complex.

When r<1, the series converges: Snn→∞→a1-r

When r>1, the series diverges.

When r=1, if r=1, Sn grows to infinity. When r=-1, the partial sums oscillate between ±a. More generally, when r=ej2πτ for any integer τ, the series will circulate indefinitely with a period of τ.

Derivation of partial sum closed form:

Sn=k=0nark

rSn=k=1n+1ark

Sn-rSn=Sn1-r=a-arn+1

Sn=a1-rn+11-r, r≠1an+1, r=1

Derivation of infinite series closed form for r<1:

Snn→∞=a1-rn+11-rn→∞=a1-r

### P series

https://en.wikipedia.org/wiki/Convergence_tests#p-series_test

n=k∞1np, k>0

This converges if p>1.

Harmonic series (p=1,k=1) diverges.

Basel problem (p=2,k=1) converges to π26.

### Types of convergence

Uniform convergence, pointwise convergence, mean-square convergence

Absolute convergence, squared convergence

## Finding polynomial roots

For performing stability analysis of systems.

For 2nd-order polynomial, you can use the quadratic formula:

z=-b±b2-4ac2a

For higher-order, you will likely want to use numerical methods.

A special Nth-degree polynomial that occurs in discrete-time systems:

Ps=sN-1

To find the roots, Ps=0=sN-1.

zN=1=ej2πk

z=ej2πkN,k=0,1,…,N-1

The roots equally spaced around the unit circle in the complex plane at separation angle 2πN.

If Ps=sN-aN where a is a real number, then the roots instead have magnitude a.

z=aej2πkN

## Triangle inequality

# Intro to continuous-time signals and systems

## Complex exponential

General complex exponential (4 parameters):

xt=Aeγt

γ=-α+jw0

A=Aejϕ

xt=Ae-αtejw0t+ϕ

The general complex exponential encompasses several important and special cases.

Real exponential: A and γ are real.

γ=-α, A=A

xt=Ae-αt,-∞<t<∞

A controls the amplitude and α controls the decay rate.

In practice, we usually work with real exponentials that “turn on” after a certain point. We model this with the unit step function ut.

xt=Ae-αtut

Complex and real sinusoids: A is complex and γ is imaginary.

γ=jw0, A=Aejϕ

xt=Aejϕejw0t=Acosw0t+ϕ+jsinw0t+ϕ

xt=xret+jximt

xret=Acosw0t+ϕ

ximt=Asinw0t+ϕ

A is the signal amplitude, w0 is frequency in rad/s, and ϕ is the phase.

w0=2πf0, f0 is frequency in Hz. T0=1f0 is period in seconds.

Damped complex and real sinusoids: A and γ are complex.

xt=Ae-αtejw0t+ϕut=xret+jximt

xret=Ae-αtcosw0t+ϕut

ximt=Ae-αtsinw0t+ϕut

α>0 for damping.

## Singularity and other special signals

These signals are piecewise continuous – a formal derivative does not exist everywhere, and the signal may contain jumps.

Rectangle pulse and triangle pulse:

Πtτ=1, t≤τ20, t>τ2

Λtτ=1-tτ,t≤τ0, t>τ

The width of the rectangle pulse is τ, but the width of the triangle pulse is 2τ. This is not an accident – the triangle pulse is the result of the rectangle pulse convolved with itself.

Unit impulse (Dirac delta):

Test waveform to find the impulse response of systems.

You can define this signal only in an operational sense, as in how it behaves. It is a spike with zero width and unity area, i.e.

- t1t2δt-t0dt=1, t1<t0<t2

- δt-t0=0, t≠t0

It’s a function with unit area located at t=t0.

Sifting property of the Dirac delta:

-∞∞xtδt-t0dt=xt0

Approximation of Dirac delta:

δε=12εΠt2ε

limε→0δεt→δt

Dirac delta is drawn as a vertical line with an arrow at the top, with height equal to area.

Unit step function:

Model signals with on/off gates.

ut=-∞tδλdλ=0, t<01, t>0,undefined, t=0

δt=dutdt

Derivative of rectangle pulse:

dΠtτdt=δt+τ2-δt-τ2

For derivative of triangle pulse, ignore the discontinuities at -τ,0,τ and define the derivative elsewhere.

dΛtτdt=1τ,-τ<t<0-1τ,0<t<τ0, otherwise

A real-world step function doesn’t suddenly jump from 0 to 1 – it smoothly transitions over a period of time, which you can see when you zoom in. When you zoom out, it looks like a true mathematical step function. When you differentiate the real-world step, its derivative is defined everywhere, and the result is a pulse-like signal that looks like an impulse when viewed from a distance.

## Types of signals

Deterministic vs. random:

A deterministic signal is a completely specified function of time.

A random signal takes on values by chance according to a probabilistic model.

For example, xt=Acos2πf0t+ϕ is deterministic. However, if you make any of the parameters A, f0,ϕ random, then it becomes a random signal.

Periodic vs. aperiodic:

A periodic signal satisfies xt=xt+T0, where T0 is the period of the signal.

xt=Acos2πf0t+ϕ=Acos2πf0t+ϕ+2πk=Acos2πf0t+k+ϕ=Acos2πf0t+kf0+ϕ

k is an integer. For k=1,

xt=Acos2πf0t+1f0+ϕ→T0=1f0

Power and energy signals:

A signal has power and energy. Energy (joules) may be finite or infinite; power (watts) may be zero, finite, or infinite.

Circuit theory: P=VI=V2R=I2R

When V or I are functions of time (signals), then instantaneous power is a function of time.

pt=vtit=v2tR=i2tR

In signals and systems, we typically normalize R=1Ω.

pt=v2t=i2t,R=1

Average power and energy are defined as

P=limT→∞12T-TTxt2dt=xt2

E=limT→∞-TTxt2dt=-∞∞xt2dt

2 is for complex signals.

For a zero-mean signal, power is equivalent to variance.

For a periodic signal, power becomes

Pperiodic=1T0T0xt2dt

For a power signal, 0<P<∞ and E→∞.

For an energy signal, 0<E<∞ and P→0.

For signals that are neither (unbounded power and energy), P→∞.

Mathematically, a signal can have infinite power but this isn’t a practical reality. Infinite energy usually means signal duration is infinite, so it makes more sense to deal with power rather than energy.

Single real sinusoid: power signal with P=A22 and E→∞.

xt=Acos2πf0t+ϕ,-∞<t<∞

P=1T00T0xt2dt=1T0A220T01+cos2π2f0t+2ϕdt=A22T00T01dt=A22

0T0cos2π2f0t+2ϕdt=0 because you’re integrating over two full periods.

Two real sinusoids at different frequencies: power signal with P=A122+A222 and E→∞. xt does not need to be periodic.

xt=A1cos2πf1t+ϕ1+A2cos2πf2t+ϕ2

xt2=x2t=A1221+cos2π2f1t+2ϕ1+A2221+cos2π2f2t+2ϕ2+A1A2cos2πf1+f2t+ϕ1+ϕ2+A1A2cos2πf1-f2t+ϕ1-ϕ2

In the equation of power, as T→∞, the cosine terms are vanishingly small (the constant terms dominate). So

P=limT→∞12T-TTxt2dt→A122+A222

For xt to be periodic, T1=1f1 and T2=1f2 must be commensurate – you need to be able to find integers N1 and N2 s.t.

N1T1=N2T2→T1T2=f2f1=N2N1

The ratio of the two periods/frequencies must be a rational number, and the fundamental period is T0=N1T1=N2T2. In algebraic terms, T0 is the least common multiple (LCM) of the periods or f0 is the greatest common divisor (GCD) of the frequencies.

T0=LCMT1,T2

f0=GCDf1,f2

Periodicity among multiple sinusoids is essential to Fourier series.

In short, sinusoidal signals are power signals. For K sinusoids at distinct frequencies, P=12i=1KAk2. If the sinusoids have the same frequency, you need to combine them using phasor addition before calculating power.

Real exponential:

xt=Ae-αtut

Energy signal if α>0, power signal if α=0, and neither if α<0.

Even and odd signals:

Symmetry along the time axis relative to the origin, t=0.

Even: x-t=xt

Odd: x-t=-xt

(Or neither.)

cosw0t is even, sinw0t is odd.

## Signal transformations

Time shifting: xt→xt-t0

Time reversal: xt→x-t

Shift and reverse: xt→xt0-t

xt→xt-t0→x-t-t0=xt0-t

xt-t0 shifts xt by t0, and then x-t-t0 reverses it around the point t-t0=0, or t=t0. (The origin is essentially shifted from 0 to t0.)

If xt is a finite-duration signal bounded by ta and tb, then xt0-t is bounded by t0-tb and t0-ta. In the example below,

ta=-1, tb=2

t0-tb=5, t0-ta=8

Superimposing signals: xt=x1t+x2t+…=k=1Kxkt

## System properties

Linear and nonlinear:

A system is linear if superposition holds.

x1t→T→y1t=Tx1

x2t→T→y2t=Tx2

If the system is linear, then

ax1t+bx2t→T→ay1t+by2t

In general,

yt=Tk=1Kakxkt=k=1Kakykt

If this isn’t true, then the system is nonlinear.

For example, you want a karaoke system to be linear so that the singer’s voice and the track can merge together in the audio amplifier without distortion. On the other hand, hard rock guitarists may intentionally send their signal through a nonlinear amplifier to get some distortion.

Time-invariant and time-varying:

A system is time-invariant if its properties or characteristics don’t change with time:

xt→T→yt=Tx

xt-t0→T→yt-t0

A system that doesn’t obey this condition is time-varying. For example, twisting the volume control on your car stereo means that the gain of your system is time varying.

Noise-removing filters are typically designed to be time-invariant – assuming the noise characteristics and desired signal characteristics are fixed, the filter design should be time-invariant.

A time-varying filter – aka an adaptive filter – is needed when the noise signal characteristics change over time, like in noise-canceling headphones.

Causal and noncausal:

A system is causal if all output values, yt0, depend only on input values xt for t≤t0. The current output depends only on past and current inputs.

A non-causal system can use future input values to generate the current output – essentially, it can predict the future because it’s anticipating future inputs. A causal system is also called non-anticipative.

A non-causal system is more of a mathematical concept than a practical reality (and is physically impossible for real-time processing). With discrete-time signals and systems, it’s possible to store a signal in memory and then process it later using a non-causal system, but in reality, you’re still using past input values because you’re working with a recorded signal.

Mathematical definition 1: If x1t=x2t ∀ t<t0, then y1t=y2t ∀ t<t0.

Mathematical definition 2: If ht is the impulse response of the system H, then H is causal if and only if ht=0 ∀ t<0.

Memory and memoryless:

In a memoryless system, each output yt depends only on the current input xt.

A system with memory (a filter) uses current and past values of the input to form the current output. For example, systems described by linear constant coefficient differential/difference equations, or an electronic circuit composed of resistors, capacitors, and inductors. The capacitors and inductors are the memory elements; a resistor-only network has no memory.

Bounded-input bounded-output (BIBO):

A signal that is bounded has magnitude less than infinity over all time. A signal xt is bounded if there exists a constant Bx>0 s.t. xt≤Bx<∞ ∀ t.

A system is BIBO-stable if every bounded input produces a bounded output.

Choosing linear and time-invariant systems (LTI):

Typically, we’re most interested in LTI systems because they are the easiest to analyze for system performance.

Example systems:

yt=Tx=2xt+5

ax1+bx2→2ax1+2bx2+10

ay1+by2=a2x1+5+b2x2+5=2ax1+2bx2+5a+b

Tax1+bx2≠ay1+by2, so the system is nonlinear. The system is linear if there is no output offset.

Txt-t0=2xt-t0+5=yt-t0, so the system is time-invariant. Also by inspection, none of the system parameters – 2 and 5 – vary with time.

The system is causal and memoryless.

If x≤Bx, then y=2x+5≤2x+5≤2Bx+5<∞, so the system is BIBO stable.

Triangle inequality: x+y≤x+y

Next system.

yt=Tx=sin2πt⋅xt+ut-2

Like the previous system, this is nonlinear. It is conditionally linear when the step function is off (t-2<0).

This system is time-varying because the system coefficients –  sin2πt and ut-2 – are functions of time.

This system is causal and memoryless.

y=sin2πt⋅xt+ut-2≤xt+1=Bx+1<∞

This system is BIBO stable.

# Intro to discrete-time signals and systems

## Complex exponential

xn=Aγn

A=Aejϕ

γ=αejw0

xn=Aαnejw0n+ϕ

The equivalent continuous-time signal was written slightly differently: xt=Ae-αtejw0t+ϕ. But we could have written it as xt=Ae-αtejw0t+ϕ.

Real exponential:

ϕ=0, w0=0→xn=Aαnun

Complex exponential:

xn=Aαnejw0n+ϕ=Aαncosw0n+ϕ+jsinw0n+ϕ

Complex sinusoid:

xn=Aejw0n+ϕ

Real sinusoid:

xn=Acosw0n+ϕ

Continuous-time vs. discrete-time frequency:

Let Ω be continuous-time frequency and ω be discrete-time frequency. Then

AcosΩt+ϕ

Acosωn+ϕ

Ω is in radians/second; ω is in radians/sample. They are not the same.

In practice, discrete-time signals are generated by uniformly sampling continuous-time signals at a sampling rate of fs or a sampling period of T=1fs.

t becomes n:

t→nT

xn=AcosΩt+ϕt→nT=AcosΩnT+ϕ=?Acosωn+ϕ

ω=ΩT=Ωfs=2πffs

Where -fs2<f<fs2 for no aliasing.

## Special signals

Unit impulse:

δn=1, n=00, otherwise

Any sequence can be expressed as a linear combination of time-shifted impulses, which is important to convolution:

xn=k=-∞∞xkδn-k

δn-k is nonzero only at n=k.

Unit step:

un=1,n≥00,otherwise

Relationship with unit impulse:

un=k=-∞nδk=alsok=0∞δn-k

δn=un-un-1

A rectangle pulse of length L can be expressed as rn=un-un-L.

Window functions:

yn=xnwn

wn is the window function.

Pulse shapes:

Pulse shapes pn create a digital communication waveform that’s bandwidth or spectrally efficient, e.g.

xn=kdkpn-kNs

dk are data bits (±1) and Ns is the duration of each bit in samples. Popular pulse shapes: rectangle, half-sine, raised cosine, square-root raised cosine.

## Types of signals

Like CT, DT signals may be deterministic or random (common distributions include Gaussian and uniform).

Periodic and aperiodic:

If there exists N0 s.t. xn=xn+N0, then xn is periodic with period N0.

In DT, a sinusoidal signal isn’t always periodic. To be periodic,

Acosw0n=Acosw0n+N0=Acosw0n+w0N0

Since sinusoids are mod 2π,

Acosw0n+w0N0=Acosw0n+2πk

w0N0=2πk→w0=2πkN0

Therefore, a sinusoid is periodic only if w0 can be written as 2πkN0 where k and N0 are integers. For multiple added sinusoids to be periodic, you need to find a common N0 for all of them.

Again because sinusoids are mod 2π,

Acos2πkN0n=Acos2πkN0n+2πmn=Acos2πN0k+mN0n

That is, a sinusoid of frequency 2πkN0 is indistinguishable from a sinusoid of frequency 2πN0k+mN0. Therefore, the unique sinusoids that are periodic with period N0 have frequencies given by

2πkN0,k=0,1,…N0-1

These are called distinguishable frequencies. They are distinct from all other frequencies that result in a sinusoidal sequence having period N0 and lie on the fundamental frequency interval w0∈0,2π.

In CT, oscillation rate increases as Ω increases. In DT, oscillation rate increases as ω increases for ω∈0,π and decreases as ω increases for ω∈π,2π.

Power and energy signals:

Ex=limN→∞n=-NNxn2

Px=limN→∞12N+1n=-NNxn2

Energy signal: Ex<∞,Px=0

Power signal: Px<∞,Ex→∞

Neither: Ex→∞,Px→∞

An aperiodic signal of finite duration is an energy signal, and a sinusoidal signal is a power signal.

xn=Acosw0n+ϕ

Px=limN→∞12N+1n=-NNA221+cos2w0n+2ϕ→A22

For a zero-mean signal, power is equivalent to variance.

## System properties

Linearity condition:

yn=Tk=1Kakxkn=k=1Kakykn

Time-invariance condition:

xn→T→yn

xn-n0→T→yn-n0

In CT, time-varying behavior can be due to uncontrollable like environmental conditions.

In DT, adaptive filters with time-varying attributes can optimize system performance. For example, the channel between your cell phone and the base station changes as you move, so your cell phone adapts to the environment by changing system attributes. This is part of adaptive signal processing.

Causality condition:

A system is causal if yn0 depends only on xn where n≤n0. The present output depends only on past and present input values.

A system is causal if x1n=x2n ∀ n<n0 and y1n=y2n ∀ n<n0.

A system is causal if hn=0 ∀ n<0.

Memory:

A system is memoryless if yn depends only on xn. Otherwise, it has memory.

BIBO stability:

A system is BIBO stable if and only if every bounded input produces a bounded output.

# CT LTI systems

## Impulse response and convolution integral

If you know the impulse response for an LTI system, you can calculate the output of that system for any input.

The impulse response ht of an LTI system is the output of that system when the input is δt, i.e. ht=Lδt. The system must be at rest when it receives the impulse.

By the sifting property, any signal xt can be represented as

xt=-∞∞xλδt-λdλ

For an LTI system,

yt=-∞∞xλht-λdλ

yt=-∞∞hλxt-λdλ

Convolution obeys standard commutative, associative, and distributive algebraic properties.

Commutative:

yt=xt*ht=ht*xt

Associative (also known as series or cascading connection):

xt*h1t*h2t=xt*h1t*h2t

hseries=h1t*h2t

Distributive (aka parallel connection):

x1t+x2t*ht=x1t*ht+x2t*ht

xt*h1t+xt*h2t=xt*h1t+h2t

hparallel=h1t+h2t

Useful identities:

δt*xt-t0=xt-t0

δt-t0*xt=xt-t0

δt-t1*xt-t2=xt-t1-t2

δt-t1*δt-t2=δt-t1-t2

Convolution examples with finite extent signals:

If xt is bounded by t1, t2 and ht is bounded by t3, t4, then yt=xt*ht is bounded by the “support interval” (the interval where yt≠0) t1+t3,t2+t4 and the duration of yt is Tx+Th, the durations of xt and ht.

Exponential impulse response:

A system with an exponential impulse response, e-at or e-tτ, models a first-order lowpass filter. This impulse response distorts the input pulse. When input pulses are sent at too high of a rate, this distortion can cause errors at the receiver (ISI). A rule of thumb is that the input pulse duration should be 10 times the time constant, which means that at the center of the input pulse, the tail from the previous pulse has decayed to e-5≈0.007.

Consider rectangular input pulses of duration T=1, 5, 10 and τ=1. The vertical dashed lines represent the centers of the pulses.

## Step response

Step response is the response of the system at-rest to the unit step.

yst=ut*ht

ut*ht=-∞∞hλut-λdλ=-∞thλdλ

That is, the step response is the integral of the impulse response.

On the flip side, differentiating the step response gives the impulse response:

ddtyst=ht

## BIBO stability and causality

An LTI system is BIBO stable if -∞∞htdt<∞.

An LTI system is causal if ht=0 for t<0.

# DT LTI systems

## Impulse response and convolution sum

DT LTI system is also known as digital filter.

The impulse response of a DT LTI system, hn, is the output produced by the at-rest system when given the input xn=δn.

Any sequence xn can be expressed as

xn=k=-∞∞xkδn-k

For an LTI system,

yn=k=-∞∞xkhn-k

yn=k=-∞∞hkxn-k

yn=xn*hn=hn*xn

Convolution is commutative:

xn*hn=hn*xn

Associative (cascaded systems):

xn*h1n*h2n=xn*h1n*h2n

hseries=h1n*h2n

Distributive (parallel systems):

x1n+x2n*hn=x1n*hn+x2n*hn

xn*h1n+xn*h2n=xn*h1n+h2n

hparallel=h1n+h2n

Useful convolutions:

δn-n0*xn=xn-n0

δn-n0*xn-n1=xn-n1-n0

Shift + reverse transformation for convolution (same as CT):

Support intervals:

If xn is bounded by n1, n2 and hn is bounded by n3,n4, then yn=xn*hn is bounded by n1+n3,n2+n4 and its duration is Nx+Nh-1, where Nx and Nh are the durations of x and h.

This easily extends to semi-infinite duration sequences.

E.g. xn has support 0,∞ and hn has support 5, 20, then yn has support 0+5,∞+20=5, ∞ and duration ∞.

E.g. xn has support -∞,∞ and hn has support 0, 10, then yn has support -∞,∞ and duration ∞.

Example: unit step * decaying exponential

Consider two semi-infinite duration sequences: xn=un and hn=anun where a is real with magnitude less than 1.

yn=x*h=kakukun-k

yn, n≥0=k=0nak→yn=k=0nakun=1-an+11-aun

By geometric series.

limn→∞yn=11-a

Example: rectangular pulse * decaying exponential

Let xn=un-un-N and hn=anun. By LTI and using the previous example,

yn=1-an+11-aun-1-an-N+11-aun-N

yn=1-an+1a-1, n∈0, N-1an-N+11-aN1-a, n≥N0, otherwise

As in the CT case, a system with a decaying exponential impulse response is a lowpass filter – it slows down the edges of the rectangular pulse. The system charges up during n∈0,N-1 and discharges during n≥N.

One case where this “slowing down” may be desirable: in the action of pressing a garage door opener, you need to hold down the button for some duration of time to make sure the charging up reaches the triggering threshold. This reduces the chance of false triggering.

## Step response

The step response of an LTI system at rest, sn, is its output in response to the input step, xn=un.

sn=u*h=khkun-k=k=-∞nhk

In other words, the step response is the accumulation of the impulse response.

For example, if hn=anun,

sn=k=-∞nakuk=k=0nak=1-an+11-aun

The inverse is given by

hn=k=-∞nhk-k=-∞n-1hk=sn-sn-1

The impulse response is the “differencing” of the step response.

## BIBO stability

An LTI system is BIBO stable if

n=-∞∞hn<∞

Proof:

yn=kxn-khk≤kxn-khk≤Bxkhk

Then yn<∞ if Bx<∞ and khk<∞.

Consider hn=anun.

nanun=n=0∞an=11-a, a<1∞, a≥1

The system is BIBO stable only if a<1. Note that by this definition, un is not BIBO stable, but it’s still a useful building block.

## Causality

An LTI system is causal if hn=0 for n<0.

# LCCDE sinusoidal steady-state analysis

LCCDE are a special class of LTI systems:

- LCC differential equations for CT systems

- LCC difference equations for DT systems

LCC difference equations are relatively new to signals and systems; they are important for efficient DSP.

## CT LTI sinusoidal steady-state response

To solve LCC differential equations for the general solution, we need Laplace transforms. For now, we will focus on the sinusoidal steady-state solution.

The output of a BIBO-stable system, in response to a sinusoidal input as time goes to infinity, is a sinusoid of the same frequency.

The impulse and step responses of causal and stable LTI systems will usually include transient terms of the form Kie-ait. Transient terms die out as t→∞, so only the steady-state response is left.

Steady-state analysis is finding the output when the input is

xt=A ej2πf0t+ϕ, -∞<t<∞

For a causal and stable system,

yt=λhλxt-λdλ=λhλA ej2πf0t-λ+ϕdλ=λhλA ej2πf0t+ϕe-j2πf0λdλ=xtλhλe-j2πf0λdλ=Hf0xt

The output of the system in response to the complex sinusoid is the same sinusoid multiplied by Hf0, the frequency response of the system at f0.

The frequency response of the system is

Hf=λhλe-j2πfλdλ=Hfej∠Hf

Hf is the Fourier transform of the impulse response ht.

If ht is real, then

H-f=λhλej2πfλdλ=λhλe-j2πfλdλ*=H*f=Hfe-j∠Hf

Real sinusoid, real ht:

xt=Acos2πf0t+ϕ=A2ej2πf0t+ϕ+e-j2πf0t+ϕ

yt=Hf0A2ej2πf0t+ϕ+H-f0A2e-j2πf0t+ϕ=Hf0⋅A2ej2πf0t+ϕ+∠Hf0+Hf0A2e-j2πf0t+ϕ+∠Hf0=AHf0cos2πf0t+ϕ+∠Hf0

## LCC differential equations

LCC differential equations have been around for a long time in electrical engineering, mechanical, chemical, biological, and other sciences. They are used to model time-varying signals in LTI systems, like a circuit composed of resistors, capacitors, and inductors.

Nth-order LCCDE:

k=0Nakdkytdtk=k=0Mbkdkxtdtk

k=0Nakykt=k=0Mbkxkt

xt and yt are the input and output, and N is the highest derivative in the output.

The coefficients ak and bk completely describe the system.

When the input and its derivatives are nonzero, this is a nonhomogeneous differential equation. When the input and its derivatives are zero, then it is a homogeneous differential equation, i.e.

k=0Nakykt=0

Frequency response:

xt=ej2πf0t→yt=Hf0ej2πf0t

xkt=j2πf0kej2πf0t

ykt=Hf0j2πf0kej2πf0t

Hf0ej2πf0tk=0Nakj2πf0k=ej2πf0tk=0Mbkj2πf0k→Hf0=k=0Mbkj2πf0kk=0Nakj2πf0k

bk, corresponding to x, are the numerator coefficients.

ak, corresponding to y, are the denominator coefficients.

k indicates the derivative order.

Example: y''t+2y't+2yt=2xt

The coefficients are bk=2, ak=1,2,2. You can use scipy.signal.freqs to calculate the frequency response. For a 1Hz real sinusoid, you can calculate the steady-state output using the frequency response, but you can also simulate the full response using scipy.signal.lsim, which includes the transient plus steady state response starting from t=0.

## DT LTI sinusoidal steady-state response

The general output of DT LTI systems will contain transient terms of the form Kiain. For table systems, ai<1, so the transients die as n→∞, leaving only the steady-state response.

xn=Aejw0n+ϕ

yn=khkxn-k=khkAejw0n+ϕe-jw0k=khke-jw0kxn=Hejw0xn

The frequency response of the system is

Hejw=nhne-jwn=Hejwej∠Hejw

The frequency response is periodic with period 2π:

Hejw+2π=nhne-jwne-j2πn=nhne-jwn=Hejw

Real sinusoid, real hn:

xn=Acosw0n+ϕ

yn=AHejw0cos(w0n+ϕ+∠Hejw0

## LCC difference equations

Convolution sum may require an infinite number of calculations due to the doubly infinite sum, but LCC difference equations make DT LTI systems possible.

Nth-order LCCDE:

k=0Nakyn-k=k=0Mbkxn-k

N is the number of feedback terms (also, the system order), and M is the number of feedforward terms. The system is completely defined by the coefficient sets ak and bk.

In continuous time, the LCC differential equation represents a circuit, but in discrete time, the LCC difference equation is exactly the system.

Generally, a0=1 (or the coefficients are normalized by a0). Then

yn=k=0Mbkxn-k-k=1Nakyn-k

The general form is an IIR filter.

When ak=0 for k>0, then it becomes an FIR filter,

yn=k=0Mbkxn-k

Note that this is the same form as DT convolution, where the impulse response is

hk=bk,0≤k≤M0, otherwise

Example: accumulator system

An accumulator system is defined by hn=un.

yn=kxkun-k=k=-∞nxk

This is an IIR, which means you need recursion (LCCDE) to represent it in the real world.

yn-1=k=-∞n-1xk

yn-yn-1=k=-∞nxk-k=-∞n-1xk=xn

yn=yn-1+xn

N=1, a0=1, a1=-1, M=0, b0=1

Using recursion to find the impulse response:

Recursion works for solving simple systems (z-transforms will be necessary for complex systems).

yn=ayn-1+xn, a<1

Let xn=K⋅un, a scaled step sequence. Let the initial condition be y-1=c.

The output is found iteratively, calculating yn for each n starting at 0.

y0=ay-1+x0=ac+K

y1=ay0+x1=aac+K+K=a2c+a+1K

y2=aa2c+a+1K+K=a3c+a2+a+1K

yn=an+1c+Kk=0nak=an+1c+K1-an+11-a, n≥0

For c=0 and K=1,

yn=1-an+11-aun

This is the step response of the at-rest system defined by hn=anun.

You can also find the impulse response by setting the initial conditions to zero and letting xn=δn.

y0=δ0=1

y1=a

y2=a2

yn=hn=anun

Frequency response:

yn=Hejw0ejw0n

k=0Nakyn-k=k=0Mbkxn-k

xn-k=ejw0ne-jw0k

yn-k= Hejw0ejw0ne-jw0k

Hejw0k=0Nakejw0ne-jw0k=k=0Mbkejw0ne-jw0k→

Hejw=k=0Mbke-jwkk=0Nake-jwk

k indicates the delay order.

Example: FIR filter, M=2

b0,b1,b2=1,2,3

xn=2cosπ5n

Hejπ5=k=02bke-jπk5

# Fourier series of periodic CT signals

## Frequency domain and line spectra plots

Frequency domain representation is defined by the three parameters of a sinusoid: amplitude, phase, and frequency.

Complex sinusoid:

xt=Aej2πf0t+ϕ, -∞<t<∞

xt has a spectral amplitude of A at f0 and spectral phase of ϕ at f0. Characterized by f0,Aejϕ, aka frequency-amplitude/phase pair.

Real sinusoid:

xt=Acos2πf0t+ϕ=A2ej2πf0t+ϕ+e-j2πf0t+ϕ

f0,A2ejϕ, -f0,A2e-jϕ

For K real sinusoids,

xt=A0+k=1KAkcos2πfkt+ϕk=A0+k=1KAk2ejϕkej2πfkt+k=-K-1Ak2e-jϕke-j2πfkt=A0+k=1KXkej2πfkt+k=-K-1Xke-j2πfkt

For real signals,

X-k=Xk*

The constant (DC) term is characterized by 0, A0. If A0<0, then 0, A0ejπ.

Line spectra plot = plot amplitude or phase against frequency as vertical lines. (Plot f, Aejϕ).

- Two-sided plot shows both positive and negative frequencies

- Only for real signals: one-sided plot shows only nonnegative frequencies. Compared to two-sided plot, amplitudes are doubled (except for DC). Acos2πf0+ϕ is characterized by f0,Aejϕ.

Symmetry of real signals:

When xt is real, the expansion into complex sinusoids always results in conjugate symmetry of Xk coefficients, X-k=Xk*.

- Double-sided amplitude spectrum has even symmetry: X-k=Xk

- Double-sided phase spectrum has odd symmetry: ∠X-k=-∠Xk

## Fourier series

A signal xt that is a sum of sinusoids may be periodic or aperiodic. If the greatest common divisor (GCD) of the sinusoid frequencies exists, then xt is periodic with period equal to 1/GCD; otherwise, xt is aperiodic.

When a signal is periodic, every frequency in the signal is harmonically related to the fundamental frequency f0 (which is equal to the GCD).

xt=k=1KAkcos2πfkt+ϕk

That is, fk=nf0, every frequency is an integer multiple of f0, and the signal is periodic with period T0=1f0.

Fourier series:

When a signal is periodic, you can convert between its time-domain and frequency-domain representation using Fourier series. Let f0 be the fundamental frequency and T0 be its period. The Fourier coefficients, Xk, are the frequency-amplitude/phase pairs discussed earlier – if you can directly write xt as a sum of sinusoids, then you do not need to use the analysis equation to find Xk.

Synthesis equation (frequency  time):

xt=k=-∞∞Xkej2πkf0t

Analysis equation (time  frequency):

Xk=1T0T0xte-j2πkf0tdt

DC term is average waveform value:

X0=1T0T0xtdt

### Deriving the analysis equation from the synthesis equation

The first step is to establish the orthogonality of fkt=ej2πkfot when integrated over a T0 interval.

Orthogonality means that the dot (inner) product of two functions is zero.

Dot product for complex functions is defined as

fkt⋅flt=T0fktfl*tdt

T0ej2πk-lf0tdt=ej2πk-lf0tj2πk-lf00T0=ej2πk-l-1j2πk-lf0

This evaluates to 0 when k≠l. When k=l,

limk'→0ej2πk'-1j2πk'f0=limk'→0j2πej2πk'j2πf0=1f0=T0

Therefore, fkt are orthogonal when k≠l.

fkt⋅flt=0, k≠lT0, k=l

With this, we can derive the analysis equation, starting from the synthesis equation:

xt=l=-∞∞Xlej2πlf0t

xte-j2πkf0t=e-j2πkf0tl=-∞∞Xlej2πlf0t

T0xte-j2πkf0tdt=T0e-j2πkf0tl=-∞∞Xlej2πlf0tdt

T0xte-j2πkf0tdt=T0Xk

Xk=1T0T0xte-j2πkf0tdt

### Gibbs phenomenon/ringing

xt=k=-NNXkej2πkf0t

For signals with jump discontinuities (like a square wave), the synthesis equation does NOT converge to the original signal in a point-by-point sense. At the discontinuities, it converges to the average of the left- and right-hand limits; for example, for a square wave that jumps between ±A, the synthesis equation converges to 0.

Furthermore, the “reconstructed” signal oscillates around the discontinuity with a peak overshoot of around 9%. Increasing N does not reduce the overshoot; it only increases oscillation frequency.

### Pulse train/square wave

Periodic pulse train over one period:

xt=A⋅Πt-τ2τ

Xk=1T0T0xte-j2πkf0tdt=AT00τe-j2πkf0t=AT0e-j2πkf0t-j2πkf00τ=AT01-e-j2πkf0τj2πkf0=AT0ejπkf0τ-e-jπkf0τj2πkf0e-jπkf0τ=AτT0j2sinπkf0τj2πkf0τe-jπkf0τ=AτT0sinπkf0τπkf0τe-jπkf0τ→

Xk=AτT0sinckf0τ⋅e-jπkf0τ

The Fourier coefficients of a periodic pulse train follow a sinc function. (sinc of 0 is 1.)

For a square wave,

τ=T02→

Xk=A2sinck2e-jπk2=A2sinπ2kπ2ke-jπk2=Aπksinπ2ke-jπk2

For k even, sinπ2k=0. For k odd, sinπ2ke-jπk2=e-jπ2=1j.

Xk=A2,k=0Ajπk, k odd0, otherwise

Example rectangular pulse train:

A=1, f0=1,τT0=18

Xk=18sinck8e-jπ8k

Remember: the line spectra sit at multiples of the fundamental frequency, or kfo.

Xk are zero when sinπkfoτ=0, or πkfoτ=nπ, or kfoτ=n. (n is an integer.) Thus, the nulls/sidelobes are defined by kfo=nτ or k=nfoτ=nT0τ.

If you keep τ constant, the nulls are always at ±1τ, ±2τ, etc. regardless of To.

If you keep To constant, decreasing τ pushes out the nulls.

For square waves, Toτ=2, so k=2n  even harmonics are null.

Because of e-jπkfoτ term, the slope of the phase is equal to -πτ w.r.t. frequency kfo. At each null, the sinc term switches between ±1, which causes a phase jump of π.

Example square wave signal:

### Triangle wave

xt=2tT0, 0≤t<T022T0-tT0,T02≤t<T0

Xk=A2,k=0-2Aπ2k2, k odd0, otherwise

As with the square wave, even-order harmonics are null. Since the triangle wave is smoother, the harmonics roll off more quickly as 1k2 (compared to 1k). For the same reason, synthesizing the triangle wave using Fourier synthesis also works out much better than for the square wave.

In the plot below, harmonics are normalized to the fundamental (fundamental is at 0dB).

### Rectified sine waves

Half-wave rectified sine-wave:

xt=Asin2πf0t, sin2πf0t≥00, otherwise

Xk=Aπ,k=0-jAk4,k=±1-Aπk2-1,k even0, otherwise

Full-wave rectified sine-wave:

xt=Asin2πf0t

Xk=2Aπ,k=0-2Aπk2-1,n even0, otherwise

### Properties

xtFXk

DC level shifting and gain scaling:

yt=A+BxtFA+BX0,k=0BXk,n≠0

Time shifting:

yt=xt-t0FXke-j2πkf0t0

Symmetry properties for real signals:

- x is an even function of time, i.e. x-t=xt, then Xk are real.

- x is an odd function of time, i.e. x-t=-xt, then Xk are imaginary.

- x has odd half-wave symmetry, i.e. xt±T02=-xt, then Xk=0 for k even. Ex: square waves and triangle waves with DC = 0.

Parseval’s theorem:

Power in periodic signal xt:

P=1T0T0xt2dt=k=-∞∞Xk2

If x is real,

P=X02+2k=1∞Xk2

The power of xt is equal to the sum of the power of each component because the components are orthogonal.

1T0T0xt2dt=1T0T0kXkejkw0t2dt=1T0T0kXkejkw0tkXkejkw0t*dt=1T0T0kXkejkw0tkXk*e-jkw0tdt=1T0T0klXkXl*ejkw0te-jlwotdt=1T0T0kXkXk*dt=1T0T0kXk2dt=kXk2

# CTFT

## CTFS to CTFT

Consider the periodic pulse train with fixed τ, centered around t=0. The Fourier series coefficients are given by

Xk=AτT0sinckf0τ

Remember, the nulls are defined by kfo=nτ  the nulls are always at ±1τ,±2τ,… regardless of fo.

If you let To→∞, xt becomes an aperiodic pulse. The spectral lines of the CTFS get closer together because the fundamental frequency fo→0, and in the limit, becomes a continuous spectrum.

In the plot, Xk is normalized by To:

ToXk=Aτ⋅sinckf0τ

The line spectra is a sampling of the continuous spectrum Xf=Aτ⋅sincfτ at f=kfo, where Xf is the Fourier transform of the aperiodic signal Πtτ.

Xk=1ToXff=kfo

## CTFT

For FT to exist, xt must be aperiodic and have finite energy.

Xf=Fxt=-∞∞xte-j2πftdt

xt=F-1Xf=-∞∞Xfej2πftdf

Xf is the spectrum of xt.

If xt has units of voltage, then Xf has units of VHz.

Alternative radian frequency form:

Xw=-∞∞xte-jwtdt

xt=12π-∞∞Xwejwtdw

Rectangular pulse:

xt=AΠt-toτ=A, t-to<τ20, otherwise

Xf=Aτ⋅sincfτ⋅e-j2πfto

sincx=sinπxπx

Nulls at fτ=±1, ±2,…→f=±1τ, ±2τ,…

Fourier transform is a linear operator:

Fax1t+bx2t=aFx1t+bFx2t=aX1f+bX2f

Fourier transform is complex:

Xf=ReXf+jImXf=Xfej∠Xf

Symmetry properties for real signals:

- xt is real  Xf is conjugate symmetric. Xf is even, ∠Xf is odd.

X-f=X*f

X-f=Xf

∠X-f=-∠Xf

- xt is real and even, x-t=xt  Xf is real

- xt is real and odd, x-t=-xt  Xf is imaginary

Energy spectral density with Parseval’s theorem:

Energy spectral density of energy signal xt is Xf2.

The unit of Xf is VHz, so the unit of Xf2 is V2sHz. V2s is energy, so this is energy/Hz.

Ex=-∞∞xt2dt

-∞∞xt2dt=-∞∞x*txtdt=-∞∞x*t-∞∞Xfej2πftdfdt=-∞∞Xf-∞∞x*tej2πftdtdf=-∞∞XfX*fdf→

Ex=-∞∞Xf2df

Total signal energy can be found by integrating over time or frequency.

### Properties

Linearity:

ax1t+bx2t

aX1f+bX2f

aX1w+bX2w

Time delay:

xt-t0

Xfe-j2πfto

Xwe-jwto

Time scaling:

xat

1aXfa

1aXwa

a<1: spreading in time = compressing in frequency

a>1: compressing in time = spreading in frequency

Frequency translation:

xtej2πfot

Xf-fo

Xw-wo

Modulation:

xtcos2πfot

12Xf-fo+Xf+fo

12Xw-wo+Xw+wo

xtsin2πfot

12jXf-fo-Xf+fo

12jXw-wo-Xw+wo

Convolution:

x1t*x2t

X1fX2f

X1wX2w

Multiplication:

x1tx2t

X1f*X2f

12πX1w*X2w

Differentiation:

dndt2xt

j2πfnXf

jwnXw

Integration:

-∞txτdτ

1j2πfXf+12X0δf

1jwXw+πX0δw

Parseval’s theorem:

-∞∞xt2dt

-∞∞Xf2df

12π-∞∞Xw2dw

Duality of the Fourier transform:

xtFXf

XtFx-f

If you take the Fourier transform of Xf, with f replaced by t, then you get xt with t replaced by -f.

Proof:

FXt=-∞∞Xte-j2πftdt=-∞∞Xtej2π-ftdt

This looks like the IFT (inverse Fourier transform), except t is the variable of integration and -f is the independent variable. So

-∞∞Xtej2π-ftdt=x-f

Duality examples:

- Time shift means multiplying by a complex exponential in freq domain, and freq shift means multiplying by a complex exponential in time domain

- Convolving in time equals multiplying in freq; multiplying in time equals convolving in freq

- Rectangular pulse = freq domain sinc; freq domain rectangle = time-domain sinc

Xf=Πf2W

FXt=2W⋅sinc2Wf=x-f

x-t=2W⋅sinc2Wt

Since sinc is even, xt=2W⋅sinc2Wt.

### Transform pairs

Notes:

- Triangle pulse is convolution of rectangular pulse with itself

- n repeated poles can easily be derived from convolution integral

- For DC, f and w representations are scaled differently b/c of the 2π term in the synthesis equation

- Reciprocal spreading property: a signal with long duration will have narrower spectrum, and a signal with short duration will have wider spectrum

| Signal | Time Domain | Freq Domain f | Freq Domain w |
| --- | --- | --- | --- |
| Rectangular pulse | Πtτ | τ⋅sincfτ | τ⋅sincwτ2π |
| Sinc | 2W⋅sinc2Wt | Πf2W | Πw4πW |
| Triangle pulse | Λtτ | τ⋅sinc2fτ | τ⋅sinc2wτ2π |
| Decaying exponential | e-atut a>0 | 1a+j2πf | 1a+jw |
| n repeated poles | tn-1n-1!e-atut a>0 | 1a+j2πfn | 1a+jwn |
| Double-sided decaying exponential | e-at a>0 | 2aa2+2πf2 | 2aa2+w2 |
| Impulse | Aδt | A | A |
| DC | A | Aδf | 2πAδw |
| Delayed impulse | δt-to | e-j2πfto | e-jwto |
| Complex sinusoid | ej2πfot | δf-fo | 2πδw-wo |
| Cosine | cos2πfot | 12δf-fo+δf+f0 | πδw-wo+δw+wo |
| Sine | sin2πfot | 12jδf-fo-δf-fo | πjδw-wo-δw+wo |
| Step | ut | 1j2πf+12δf | 1jw+πδw |
| Impulse train (time-domain sampling) | n=-∞∞δt-nTs | fsm=-∞∞δf-mfs | wsm=-∞∞δw-mws ws=2πfs |
| Convolution with impulse train (frequency-domain sampling) | n=-∞∞xt-nTs | fsm=-∞∞Xmfsδf-mfs | wsm=-∞∞Xmwsδw-mws |

### Fourier transforms in the limit for periodic power signals

Formally, FT requires xt to be an energy signal, but the technique called Fourier transforms in the limit allows you to take the FT of periodic power signals like sine/cosine/periodic pulse. This allows you to analyze energy signals and periodic power signals together.

First, singularity functions:

Consider the constant function xt=A ∀ t. Rewrite in the limit form:

- xt=limT→∞AΠtT

- FAΠtT=AT⋅sincfT

- -∞∞AT⋅sincfTdf=A (this is true for any T)

- Fxt=limT→∞AT⋅sincfT=Aδf

IFT:

-∞∞Aδfej2πftdf=A

Periodic signals:

A periodic signal with fundamental frequency fo can be decomposed into orthogonal frequencies:

xt=kXkej2πkfot

Xk=1ToT0xte-j2πkfotdt

Fxt=FkXkej2πkfot=kXkδf-kfo

The FT of a periodic signal is impulses scaled by the FS coefficients.

Alternatively, use the property of frequency domain sampling. Let pt be one period of xt, so

xt=nδt-nTs*pt=npt-nTs

Xf=Fnδt-nTs⋅Pf

The FS coefficients for nδt-nTs are given by

Xk=1TsTsnδt-nTse-j2πkfstdt=1Ts

Then

Fnδt-nTs=1Tskδf-kfs

Xf=1TskPkfsδf-kfs

The FT of a periodic signal is the FT of one period of the signal, sampled at multiples of the fundamental frequency.

These two interpretations are equivalent.

Example: periodic pulse train

xt=AnΠt-nTo-τ2τ

pt=AΠt-τ2τFPf=Aτ⋅sincfτ⋅e-jπfτ

Xf=Aτksinckfoτ⋅e-jπkfoτδf-kfo

### LTI systems in the frequency domain

yt=xt*ht

Fyt=Fxt⋅Fht

Yf=Xf⋅Hf

Hf=Fht is the frequency response of the LTI system. This is equivalent to the frequency response derived via sinusoidal steady-state response.

Properties of the frequency response:

If ht is real, H-f=H*f.

Energy spectral density:

Yf2=Xf⋅Hf2=Xf2Hf2

Example lowpass RC filter:

ht=1RCe-tRC⋅ut

Hf=Fht=thte-j2πftdt=1RC0∞e-tRCe-j2πftdt=1RC0∞e-t1RC+j2πfdt=1RCe-t1RC+j2πf-1RC+j2πf0∞=1RC⋅11RC+j2πf=11+j2πfRC=11+j⋅ff3dB

f3dB=12πRC=w3dB2π

w3dB=1RC

Cascaded and parallel filters:

Hcascadedf=H1fH2f

Hparallelf=H1f+H2f

Ideal filters:

Are not realizable. The impulse response for all ideal filters – lowpass, highpass, bandpass, bandstop – are nonzero for t<0, which means they are non-causal.

HLPf=G0Πf2B

hLPt=Go2B⋅sinc2Bt

HHPf=Go1-Πf2B

hHPT=Go1-2B⋅sinc2Bt

HBPf=GoΠf-fcB+Πf+fcB

hBPt=GoB⋅sincBtej2πfct+e-j2πfct=2GoB⋅sincBt⋅cos2πfct

HBSf=Go1-Πf-fcB-Πf+fcB

hBSt=Go1-2GoB⋅sincBt⋅cos2πfct

Realizable filters:

Butterworth, Chebyshev, Bessel, elliptic filters – approximations of ideal filters. Defined by LCC differential equations.

Hf=k=0Mbkj2πfkk=0Nakj2πfk

# Sampling theory

## Sampling (ADC)

Converting analog signal to digital: sample and quantize.

xt→

xst=xtst=xtnδt-nTs=nxnTsδt-nTs→

xnTs→xnQuantize to wordlength, Bxqun

Example:

xt=ej2πfot

xnTs=xnfs=ej2πfofsn

xn=ej2πfofsn=ejwon

wo=2πfofs

fo has units of Hz (cycles/second), Ωo=2πfo has units of radians/second, wo has units of radians/sample, wo2π=fofs has units of cycles/sample. fs in CT maps to 2π in DT.

In DT, frequency is periodic. wo is equivalent to wo+2πk where k is any integer.

ejwon=ejwonejk2πn=ejwo+2πkn

wo+2πk=2πfofs+2πk=2πfofs+2πkfsfs=2πfo+kfsfs=2πfeqfs

feq=fo+kfs

This means that when converting from CT to DT, CT signals at fo+kfs all appear at the same DT frequency wo. This is called aliasing; this is why we need an antialiasing filter before the ADC.

This also means the DT spectrum is periodic. A signal at frequency wo appears at wo+2πk ∀ k. In terms of CT frequency, a signal at fo appears at fo+kfs. In effect, the signal is repeated every fs.

What’s happening is you are convolving the CT spectrum with a frequency domain impulse train!

This leads to the Nyquist-Shannon sampling theorem, which says that in order to reconstruct xt from xn,

fs>2fo

The copy of the signal that lies within -fs2,fs2 is the principle alias. When fs<2fo, the principle alias is not at the original frequency because fo>fs2 and the signal aliases back onto itself at fo-kfs≠fo.

The principle alias is calculated by finding k s.t.

fprinciple=fo-kfs∈-fs2,fs2

fo∈kfs±fs2

2fo is called the Nyquist rate, and fs2 is called the Nyquist frequency or folding frequency.

## Quantization error

Quantize to B bits (round or truncate): xqun=QBxn

If Vm is the max voltage swing of the ADC, then

- Step size: Δ=2Vm2B

- Levels: -Vm to Vm-Δ

Note: the output saturates. Voltages outside of -Vm,Vm-Δ saturate.

Quantization error is the difference between the quantizer input and output:

en=QxnT-xnT

You can view e as an additive noise source (which we call quantization noise). Technically, it is noise-like since it’s functionally related to the input.

QxnT=xnT+en

For rounding, en∈-Δ2,+Δ2.

Average quantization noise power (assuming uniform distribution):

e2=-Δ2Δ2Pe⋅e2de=1Δe33-Δ2Δ2=23ΔΔ38=Δ212

Full sine wave power:

A22=Vm22

Δ212=2Vm2B212=4Vm212⋅22B

SQNR:

Vm22⋅12⋅22B4Vm2=3⋅22B-1

10log103⋅22B-1=102B-1log102+10log103=B20log102+10log1032=6.02B+1.76dB

## Frequency domain analysis

Frequency-domain convolution:

Xf*Sf=Xf*fsmδf-mfs=fsmXf-mfs

Xf-mfs are called the spectral translates of Xf, with Xf-0 being the principal translate.

If xt is bandlimited s.t. Xf=0 for f>W, then we can recover xt from its samples xn by applying a LPF, as long as fs>2W. W is the Nyquist frequency; 2W is the Nyquist rate.

If fs<2W, then the signal and its translates overlap, and you cannot recover xt.

## Reconstruction (DAC)

Two steps:

- Place signal samples, xn, back on the physical time axis, xst

- Interpolate a continuum of signal values between the sample values

This is a mixed-signal design problem.

Step 1:

xst=nxnδt-nTs

In frequency domain, this means going back to Xsf=fsmXf-mfs.

Step 2: pass xst through a zero-order hold (ZOH). ZOH models the output of a realizable DAC. As a filter, ZOH is modeled by a rectangular impulse response:

ht=Πt-Ts2Ts

Hf=Tssincffse-jπfTs

Remember, Xsf=fsmXf-mfs. We want Xf while suppressing all other images at Xf±mfs, m≠0. The ZOH partially suppresses these images because it has nulls at ±mfs.

The final step is to add a reconstruction filter to counteract the droop of the ZOH response (-3.9dB @ fs/2). The overall frequency response of the D-A conversion is Hf=HZOHfHrf or ht=hZOHt*hrt.

The ZOH only fully suppresses at 1fs, where fs is the DAC sampling rate. This means you will get significantly more distortion (from images) for frequencies farther from 0.

### Ideal interpolation

Let’s ignore “realizable” and do D-A conversion using an ideal interpolation filter given by

Hf=Πf2W

ht=2Wsinc2Wt

yt=yst*ht=ysτht-τdτ=nynδτ-nTsht-τdτ=nynδτ-nTsht-τdτ=nynht-nTs=2Wnynsinc2Wt-nTs

What this says: If you add up a bunch of CT sinc functions, scaled and time-shifted according to the DT signal, you will recover the original CT function.

yst is the sampled version of yt that includes all aliases at kfs where k is any integer.

The ideal reconstruction filter removes all images except for the principle alias (k=0) – this is why we need to ensure no aliasing during sampling.

I simulated a digital reconstruction filter:

### System design

Real interpolation filters have finite image suppression. You can increase the filter order to improve suppression, but this increases complexity. Alternatively, you can use oversampling to first separate the spectral translates. This begins in the discrete-time domain, using efficient digital filters, and ends with a high DAC rate. A high DAC rate eases the design requirements on the analog interpolation filter.

# DTFT

Xejw=n=-∞∞xne-jwn

xn=12π2πXejwejwndw

Formally, DTFT exists if xn is absolutely summable (but there are exceptions for special signals):

nxn<∞

Example: xn=anun.

nanun=n=0∞an=11-a, a<1

The DTFT only exists if a<1.

Xejw=n=0∞ane-jwn=n=0∞ae-jwn=11-ae-jw, a<1

## Relationship between CTFT and DTFT

In DTFT, the frequency variable w always appears wrapped in a complex exponential, ejw, which means the DTFT spectrum is periodic with period 2π.

Xejw+2πk=Xejwej2πk=Xejw

In practice, the DT sequence xn is commonly created by uniform sampling of the CT signal xt. This sampled waveform is

xst=nxnδt-nTs

Xsf=fsmXf-mfs

How do we relate the spectrum of xn to the spectrum of xt?

First, take the FT of xst:

Xsf=Fnxnδt-nTs=-∞∞e-j2πftnxnδt-nTsdt=nxne-j2πfnTs=nxne-j2πffsn

With a simple frequency conversion, Xsf is equal to Xejw!

w=2πffs=2πfTs

Xsff=fsw2π=nxne-jwn=Xejw

As we established above, Xsf is a periodic version of Xf. Mapping from f to w, we get

Xsff=w2πTs=fsmXf-mfsf=w2πTs=fsmXw2πTs-mTs

Xejw=fsmXw2πTs-mTs

Therefore, if xt is bandlimited to fs2, or equivalently, Xf=0 for f>fs2, and we look only at the principle alias/translate, then Xejw and Xf are equal except for frequency conversion and scaling by fs:

w∈-π,π: Xejw=fsXw2πTs

f∈-fs2,fs2: Xf=1fsXej2πffs

We can also do this for the radians/second form of the CTFT:

XΩ2πf=Xf

Xejw=fsmXΩwTs-2πmTs

XΩwTs=Xw2πTs

w=2πffs=Ωfs

w∈-π,π: Xejw=fsXΩwTs

Ω∈-πfs,πfs: XΩ=1fsXejΩfs

With these relationships, you can map CT spectrum to DT spectrum (sampling) and back (reconstruction).

## Symmetry properties for real signals

xn real  Xejw is conjugate symmetric, i.e. Xe-jw=X*ejw.

- ReXejw is even

- ImXejw is odd

- Xejw is even

- ∠Xejw is odd

xn real and even  Xejw is real, i.e. ImXejw=0.

xn real and odd  Xejw is imaginary, i.e. ReXejw=0.

Example: xn=0.8nun. Real signal, which means in freq domain that magnitude and real part are even, phase and imaginary part are odd.

Example: xn=an

a<1

xn=a-nu-n+1+anun

Fa-nu-n+1=n=-∞∞a-nu-n+1e-jwn=lal+1ulejwl+1=l=0∞aejwl+1=11-aejw-1=aejw1-aejw

Xejw=11-ae-jw+aejw1-aejw=1-a21-2acosw+a2

Since xn is real and even, Xejw is real.

Example: xn=-2δn+2+δn+1-δn-1+2δn-2

Xejw=nxne-jwn=-2ej2w+ejw-e-jw+2e-j2w=-4jsin2w+2jsinw

Since xn is real and odd, Xejw is imaginary.

## DTFT theorems and pairs

| Property | Signal | Transform |
| --- | --- | --- |
| Linearity | ax1n+bx2n | aX1ejw+bX2ejw |
| Time shift | xn-n0 | Xejwe-jwn0 |
| Frequency shift | xnejwon | Xejw-wo |
| Convolution | xn*hn | XfHf |
| Multiplication | xnwn | 12π-ππXejθWejw-θdθ |
| Reversal | x-n | X*ejw |
| Differentiation | nxn | j⋅dXejwdw |
| First difference | xn-xn-1 | 1-e-jwXejw |
| Accumulation | k=-∞nxk | 11-e-jwXejw |
| Downsample | xnM | 1Mk=0M-1XejwM-2πkM |
| Upsample | xnL,nL is an integer0, otherwise | XejwL |
| Parseval’s theorem | n=-∞∞xn2 | 12π-ππXejw2dw |

| Signal | Time | Frequency |
| --- | --- | --- |
| Impulse | δn | 1 |
| Delayed impulse | δn-n0 | e-jwn0 |
| Decaying exponential/first-order LPF | anun a<1 | 11-ae-jw |
|  | n+1anun a<1 | 11-ae-jw2 |
|  | rnsinwon-1sinwoun r<1 | 11-2rcoswoe-jw+r2e-j2w |
| Rectangular window | un-un-M | sinwM2sinw2e-jwM-12 |
| Sinc | sinwcnπn=wcπsincwcn | Πw2wc |
| Complex exponential | ejwon | 2πm=-∞∞δw-wo-2πm |
| Cosine | coswon+ϕ | πmejϕδw-wo-2πm+e-jϕδw+wo-2πm |
| DC | 1 | 2πmδw-2πm |
| Step | un | 11-e-jw+πmδw-2πm |
| Impulse train | kδn-kM | 2πMmδw-2πMm |

## Mean-square convergence and time-domain sinc

Remember, DTFT only exists if xn absolutely converges:

nxn<∞

A weaker form of convergence is mean-square convergence:

nxn2<∞

It is easier to satisfy this condition, but the DTFT may not converge pointwise in the frequency domain.

Example:

XLPejw=Πw2wc

xLPn=12π-ππXLPejwejwndw=12π-wcwcejwndw=1j2πnejwn-wcwc=sinwcnπn=wcπsincwcn ∀ n

Since the terms of xLPn are of the form 1n, the absolute sum is similar to the harmonic series, which diverges. The series 1n2 does converge, so xLPn is square summable.

This is an infinite-duration signal, but you can truncate the coefficients to get an approximation to the ideal LPF. Increasing the number of taps sharpens the filter cutoff, but the ripple amplitude is unchanged (in both passband and stopband). This can be mitigated with windowing.

## Finding DTFT in the limit

Allows impulse functions to exist in the frequency domain.

Let’s say you have a DTFT of δw-wo, -π≤w≤π. With DTFT periodicity, the full representation is

Xejw=m=-∞∞2πδw-wo-2πm

(Remember that when Xejw is a function only of ejw, the periodicity is built-in.)

xn=-ππXejwejwndw=ejwon

ejwonFm=-∞∞2πδw-wo-2πm

When wo=0,

1Fm2πδw-2πm

When xn=coswcn,

coswcnFmπδw-wc-2πm+πδw+wc-2πm

## LTI systems in the frequency domain

LTI system defined by impulse response hn.

yn=xn*hn

Yejw=XejwHejw

yn=F-1Yejw

Hejw=Fhn is the transfer function or frequency response.

Hejw=YejwXejw

xn=δn→Xejw=1→Yejw=Hejw

Energy spectral density:

Yejw2=XejwHejw2=Xejw2Hejw2

Series and parallel connections:

Hseries=H1ejwH2ejw

hseries=h1n*h2n

Hparallel=H1ejw+H2ejw

hparallel=h1n+h2n

Example:

yn-12yn-1=xn-14xn-1

Yejw-e-jw2Yejw=Xejw-e-jw4Xejw

Hejw=YX=1-14e-jw1-12e-jw=1+14e-jw⋅11-12e-jw

hn=δn+1412n-1un-1

# DFT

DFT allows us to use a computer to easily compute the frequency domain representation of a DT signal (and invert it). The mathematical definition of the DFT involves only complex number multiplication and addition – no integrals.

FFT is a set of efficient algorithms for computing the DFT.

## DFT is a sampled DTFT

DTFT finds the continuous spectrum of a DT signal that is indexed into double infinity.

DFT operates on an N-point signal and produces and N-point frequency spectrum. As long as N encompasses the entire finite-duration DT signal, the N-point spectrum is a sampled version of the DTFT spectrum.

Start with the DTFT.

Xejw=nxne-jwn

What are the values of Xejw at the discrete frequencies wk=2πkN, k=0,1,…,N-1?

Xejwk=Xk=nxne-jwkn=nxne-j2πkNn

e-j2πkNn is periodic with period N, so this is simplified to

Xk=n=0N-1xne-j2πkNn=n=0N-1xnWNkn

WN=e-j2πN

The complex weights WNk=e-j2πkN are called the twiddle factors.

The DFT is the DTFT sampled at wk=2πkN. In the z domain, the samples are on the unit circle. z=ejw gives the DTFT, and z=ejwk gives the DFT.

## Forward/inverse transforms

Xk=n=0N-1xne-j2πkNn=n=0N-1xnWNkn

k∈0,N-1

xn=1Nk=0N-1Xkej2πkNn=1Nk=0N-1XkWN-kn

n∈0, N-1

Each sum is composed of N terms, so DFT/IDFT can be computed with a finite number of complex multiplications and additions. This is not true of CTFT/DTFT, which both require integrations along a continuous variable.

Proof of invertibility, starting from the synthesis equation:

1Nk=0N-1XkWN-kn=1Nk=0N-1m=0N-1xmWNkmWN-kn=1Nm=0N-1xmk=0N-1WNkm-n

WNkm-n=e-j2πkNm-n=e-j2πm-nNk

Since e-j2πm-nNk is periodic with period N,

k=0N-1WNkm-n=0, m≠nN,m=n=Nδm-n

Then

1Nm=0N-1xmk=0N-1WNkm-n=m=0N-1xmδm-n=xn

## DFS mathematics underpin the DFT

DFS is the discrete-time version of Fourier series. xn is decomposed into periodic functions of the form e-j2πkNn, so xn is periodic with period N. Just like in CTFS, the periodic functions are harmonically related.

Even though DFT is for finite-length signals, in the underlying mathematics, the signal is periodic. The sequence of length N is repeated into double infinity, and the DFS contains N unique coefficients that are also repeated into double infinity. But a periodic signal can be perfectly represented with just one period, so the DFT stores only one period of the signal and the DFS coefficients.

This is like convolving with impulse train mδn-mN, which is why DFT is sampled version of DTFT.

The repeated sequence of length N – with its periodic extensions – can be represented as

xn=r=-∞∞xn+rN=xn mod N=xnN

The DFT is also periodic with period N.

Xk=r=-∞∞Xk+rN=Xk mod N=XkN

The two most important things about DFT:

- It is samples of the DTFT at 2πkN,k∈0,N-1

- The underlying signal is xn

It’s important to keep these two things in mind when using and plotting the DFT.

## Theorems and properties

| Property | Sequence | N-point DFT |
| --- | --- | --- |
| Linearity | ax1n+bx2n | aX1k+bX2k |
| Duality | Xn | x-k |
| Circular shift | xn-mN | WNkmXk |
| Circular shift frequency | WN-mnxn | Xk-mN |
| Circular convolution | m=0N-1x1mx2n-mN x1n*Nx2n x2n*Nx1n | X1kX2k |
| Multiplication | x1nx2n | 1Nl=0N-1X1lX2k-lN |
| Symmetry | xn real | Xk=X*N-k Xk=XN-k ∠Xk=-∠XN-k k∈0, N-1 |

Because of the periodic nature of the DFT, the time shift, frequency shift, and convolution theorems are slightly different from the DTFT. Circular convolution enables transform domain signal processing, which means we can implement a filter in the  frequency domain by multiplying our signal spectrum by the filter frequency response and returning to time domain via IDFT.

It's circular because the “underlying” signal is periodic, and DFT is a window into one period of that signal.

Another difference is that all sequence lengths must be the same (N).

Circular shift:

Circular convolution:

Yk=X1kX2k

yn=1Nk=0N-1YkWN-kn=1Nk=0N-1X1kX2kWN-kn=1Nk=0N-1m=0N-1x1mWNkmX2kWN-kn=1Nm=0N-1x1mk=0N-1WNkmX2kWN-kn=1Nm=0N-1x1m⋅N⋅x2n-mN=m=0N-1x1m⋅x2n-mN

Shorthand notation: x1n*Nx2n=x2n*Nx1n

## FFT

One of the pillars of modern signal processing – used in many different products and for many different applications.

- Easily calculates frequency response and spectra; able to provide real-time spectral analysis, e.g. electronic test equipment that displays spectra

- Transform domain filtering

From the DFT definition, the number of complex multiplications and additions required for an N-point transform is N2 and NN-1, respectively. FFT significantly reduces these numbers.

Xk=n=0N-1xnWNkn

For a single k, you have N complex multiplications and N-1 complex additions. Since you need to compute for N values of k, this becomes N2 and NN-1.

Decimation-in-time (DIT) FFT algorithm:

There are many FFT algorithms. The basic one is radix-2 DIT algorithm/radix-2 FFT, in which N must be a power of 2.

If N=2v, then you can break down the DFT computation into v stages of N2 two-point DFTs. The two-point DFT is the fundamental computational unit.

N=8 example:

Multiplications: 3*4*1=12, down from 64 (each two-point DFT only requires 1 complex multiplication)

## Application example: transform domain filtering

You can use DFT/IDFT to perform filtering entirely in the frequency domain, but you need to answer two questions:

- How do you make circular convolution perform ordinary linear convolution (convolution sum)?

- How do you handle a very long sequence xn while keeping the DFT length manageable?

Making circular convolution perform linear convolution:

If x is length L and h is length P, then the linear convolution output is length L+P-1.

In circular convolution, the input and output sequences are all length N.

To implement linear convolution using circular convolution, you need to zero-pad x and h to length N, where N≥L+P-1. (You still perform multiplication in the DFT domain to implement circular convolution.)

Using overlap and add to continuously filter sequences:

x is a sequence of possibly infinite duration and h is a length P FIR filter. Divide x into segments of finite length L and then filter each segment using the DFT/IDFT as described above. The IDFT outputs are length N≥L+P-1, so afterward, when you add the filtered sections together, you need to add them with overlap.

# Laplace transform

LT is a generalization of the CTFT:

- Exists for a wider class of signals than FT

- Well-suited to solving LCC differential equations: solves for the total solution (forced + transient) and manages nonzero initial conditions via simple algebraic manipulation, which means it’s also more user-friendly than CTFT

- Transforms signals and LTI impulse responses into the s domain, where s is a complex variable

- Impulse response in s domain is a ratio of polynomials in s

  - Denominator roots are poles

  - Numerator roots are zeros

## Two-sided (bilateral) LT

- Accepts two-sided signals, signals that extend into ±∞

- Closely related to the FT, which also accepts two-sided signals

- Can work with both causal and non-causal system models (one-sided LT cannot)

- Cannot deal with systems with nonzero initial conditions

XIIs=LIIxt=-∞∞xte-stdt

s=σ+jw=σ+j2πf

σ is the real axis variable, w is the imaginary axis variable.

Two-sided LT is equal to FT of the signal multiplied by an exponential weighting factor, which improves convergence of the LT.

XIIss=σ+jw=-∞∞xte-σ+jwtdt=-∞∞xte-σte-jwtdt=Fxte-σt

If FT exists, then FT is equal to two-sided LT with σ=0. (FT is LT evaluated on the jw – imaginary – axis.)

XIIs=jw=Fxt=Xw

### ROC

LT usually doesn’t converge over the entire s plane – the region where it converges is called the region of convergence (ROC).

Uniform convergence of the LT requires xte-st to absolutely converge:

-∞∞xte-stdt=-∞∞xte-stdt=-∞∞xte-σtdt<∞

Convergence only depends on Res=σ, which means ROC is defined by vertical boundaries in the s plane. This means the ROC may be a vertical strip, a half-plane bounded on one side by a vertical boundary, the entire plane, or doesn’t exist.

σ1→∞ and/or σ2→∞ is possible.

If xt is right-sided, meaning xt=0 for t<t1, then the ROC is a RHP (right-half plane) of the form Res>σ1.

If xt is left-sided, meaning xt=0 for t>t2, then the ROC is a LHP of the form Res<σ2.

If xt is two-sided, the ROC is the intersection of the RHP and LHP, σ1<Res<σ2 if σ1<σ2. If σ1>σ2, then the ROC doesn’t exist the the LT isn’t absolutely convergent anywhere.

Example: xat=e-atut, where a may be real or complex.

XIIs=-∞∞xte-stdt=0∞e-a+stdt=e-a+st-a+s0∞=1s+a

Res+a>0→Res>-Rea

Example: xbt=-e-atu-t, where a may be real or complex.

XIIs=-∞0-e-ate-stdt=-∞0-e-a+stdt=e-a+sta+s-∞0=1s+a

Rea+s<0→Res<-Rea

xat=- xb-t and they have the same LT. They differ only in ROC – you need to know the ROC to perform the inverse LT.

### Poles and zeros

When LT is a rational function, it takes the form NsDs where N and D are polynomials in s.

The roots of N are the zeros of XIIs and the roots of D are the poles.

Visualize NsDs as a 3D surface over the s plane with min and max values of 0 and infinity. At the zeros, the surface is tacked down to 0, and at the poles, the surface extends up to infinity.

### Stability of LTI systems

Recall that for a BIBO stable LTI system,

-∞∞htdt<∞

This is the condition for FT to exist and converge pointwise. FT of the impulse response is the frequency response of the system.

The LT of the system is known as the system function, and it’s the FT evaluated along jw axis. If ROC contains jw axis, then the system is BIBO stable.

In other words, these are all equivalent:

- ht is L1 or absolutely integrable

- FT exists and converges pointwise

- LT ROC contains jw axis

- The system is BIBO stable

Let HIIs be the system function. The poles of HIIs are singularities, which means the poles are outside of the ROC.

If the system is causal (right-sided), the ROC is the region to the right of the pole with the largest real part. This means that the poles for a causal and stable system must all lie in the LHP of the s plane, i.e., they all have negative real parts.

## One-sided (unilateral) LT

For analyzing causal systems with nonzero initial conditions and inputs applied starting at t=0. Cannot analyze signals that are nonzero for t<0.

Restricts integration interval to 0-,∞.

Lxt=Xs=0-∞xte-stdt

The results developed for the two-sided LT holds for one-sided LT as long as the signals and systems are causal. The poles of Hs must lie in the LHP for stability.

Example: xt=δt-t0, to≥0

Xs=0-∞δt-toe-stdt=e-sto

The ROC is the entire s plane since

e-sto=e-σto<∞→Res>-∞

Example: xt=ut

Xs=0-∞e-stdt=e-st-s0∞=1s

Res>0

The ROC does not include the jw axis.

Example: ht=2e-3tut+5e-2tut

Hs=2s+3+5s+2=2s+2+5s+3s+3s+2=7s+197s+3s+2

Zero @ -197, poles at -3, -2. Since these are LHP and the system is causal, the system is stable with ROC Res>-2.

### Theorems and properties

These are applicable to one-sided LT – the properties of the two-sided LT are sometimes different.

Signals and systems of interest are causal. Let xt have LT Xs with ROC Rx.

When combining LT (e.g. linearity and convolution theorems), the composite ROC may be larger than the intersection of the individual ROCs due to pole-zero cancellation.

Linearity:

ax1t+bx2tLaX1s+bX2s

ROC=at least Rx1Rx2

Delay:

xt-tout-toLe-stoXs

ROC=Rx

Differentiation:

dnxtdtnLsnXs-sn-1x0--⋯-sxn-20--xn-10-

ROC=Rx

Where xk=dkxtdtk.

Proof:

Ldxdt=0-∞dxdte-stdt

Use integration by parts.

u=e-st, dv=dx

0-∞dxdte-stdt=e-stxt0-∞+s0-∞xte-stdt=sXs-x0-

We reasonably assume that limt→∞e-stxt=0. It’s simple to extend this to higher orders of differentiation using mathematical induction.

Ld2xdt2=s2Xs-sx0--x10-

Ld3xdt3=s3Xs-s2x0--sx10--x20-

And so on.

The differentiation theorem is fundamental for solving LCC differential equations with nonzero initial conditions.

Integration:

0-∞xλdλLXss

ROC=RxRes>0

Example: ramp signal. xt=tut.

ut is the integral of δt, and tut is the integral of ut.

Since Lδt=1, Ltut=1s2.

Convolution:

x1t*x2tLX1sX2s

ROC=at least Rx1Rx2

s-shift:

xte-αtLXs+α

ROC=Rx shifted left by Reα

Example: xt=e-αtcoswot, xt=e-αtsinwot

First, look at the LT of each multipland:

Lejwot=0-∞ejwote-stdt=0-∞e-s-jwotdt=1s-jwoe-s-jwot∞0-=1s-jwo, Res>0

Lcoswot=121s-jwo+1s+jwo=ss-jwos+jwo=ss2+wo2, Res>0

Lsinwot=12j1s-jwo-1s+jwo=wos-jwos+jwo=wos2+wo2, Res>0

Le-αt=1s+α, Res>-Reα

Then, look at the LT of the product:

xt=e-αtcoswot=e-αt2ejwot+e-jwot

Xs=12⋅1s-jwo+α+12⋅1s+jwo+α=s+αs+α-jwos+α+jwo=s+αs+α2+wo2

Res>-Reα

xt=e-αtsinwot=e-αt2jejwot-e-jwot

Xs=12j1s-jwo+α-1s+jwo+α=12j⋅2jwos+α-jwos+α+jwo=wos+α-jwos+α+jwo=wos+α2+wo2

Res>-Reα

You can look at it in two ways:

- The original signal is e-αt, and you s-shift by ±jwo. This does not change the ROC of the original signal since the s-shift is imaginary. The poles shift from -α to -α±jwo.

- The original signal is e±jwot, and you s-shift by α. If α has a real component, then this shifts the ROC left by Reα. The poles shift from ±jwo to -α±jwo.

Initial value: If the limit exists,

limt→0+xt=lims→∞sXs

x must be bounded on 0,∞, and limt→0+xt must be exist.

Proof (https://en.wikipedia.org/wiki/Initial_value_theorem).

Suppose that x is bounded, and limt→0+xt=α.

Xs=0-∞xte-stdt→sXs=0-∞xte-sts⋅dt

Change of variable, τ=st.

sXs=0-∞xτse-τdτ

lims→∞sXs=0-∞x0+e-τdτ=x0+

Final value: If the limit exists,

limt→∞xt=lims→0sXs

Both limits must exist. For example, limt→0xt does not exist for xt=sint.

The poles of Xs must lie in the LHP, except for (potentially) a single pole at the origin.

This theorem is useful for determining steady-state error in control systems because you don’t need to use inverse LT to get the full time-domain response.

Proof. Start with differentiation theorem:

Lx't=0-∞x'te-stdt=sXs-x0-

lims→0sXs=lims→00-∞x'te-stdt+x0-=xt0-∞+x0-1=x∞

Example: apply final value theorem to H1s=5s+1 and H2s=4s2+4.

lims→0sH1s=lims→05ss+1=0

limt→∞h1t=limt→∞5e-tut=0

Checks out.

lims→0sH2s=lims→04ss2+4=0

limt→∞h2t=limt→∞4sin2tut→limit does not exist!

This one doesn’t work. The final value theorem does not apply because H2s contains poles on the jw axis.

### Transform pairs

| Signal | xt | Xs | ROC |
| --- | --- | --- | --- |
| Impulse | δt | 1 | all s |
| Impulse derivative | δnt=dnδtdtn | sn | all s |
| Exponential | e-atut | 1s+a | Res>-Rea |
| Repeated pole | tnn!e-atut | 1s+an+1 | Res>-Rea |
| Cosine | coswotut | ss2+wo2 | Res>0 |
| Sine | sinwotut | wos2+wo2 | Res>0 |
| Damped cosine | e-αtcoswotut | s+αs+α2+wo2 | Res>-Reα |
| Damped sine | e-αtsinwotut | wos+α2+wo2 | Res>-Reα |

### Inverse LT

Formally, ILT requires contour integration, a line integral over a closed path in the s plane. We’ll calculate the ILT using partial fraction expansion (PFE) and table lookup.

We need Xs in the form of a rational fraction.

Xs=NsDs=k=0Mbkskk=0Naksk

Xs must be a proper rational fraction, which means N>M. If it isn’t, you need to perform long division to reduce the order of the numerator.

Xs=r=0M-NKrsr+N1sDs

Distinct poles:

For simple poles (no repeats) and Xs is a proper rational, Xs takes the form

Xs=k=1NRks-pk

Find Rk (PFE coefficients) using the standard PFE residue formula.

Twin poles:

When Xs has a pole, pi, that repeats once (multiplicity of two), then

Xs=S1s-pi+S2s-pi2+k=1,k≠iNRks-pk

Find Rk using the residue formula.

Find S2=s-pi2Xss=pi.

Find S1 last.

Complex conjugate poles:

When you have a LT with a complex conjugate pole pair, the inverse transform is a linear combination of a damped cosine and a damped sine.

Xs=b1s+b0s2+a1s+a0

The poles are given by -a1±a12-4a02. For complex conjugate poles, 4a0>a12.

Xs=b1s+b0s2+a1s+a0=c1s+α+c2wos+α2+wo2

First, find α and wo from the denominator.

s2+a1s+a0=s+a122+a0-a124=s+α2+wo2

α=a12

wo=a0-a124

Then find the linear coefficients c1 and c2.

b1s+b0=c1s+α+c2wo

c1=b1

c2=b0-b1αwo

Example: Xs=s+12s2+6s+15

α=3,wo=6

c1=1, c2=96

xt=e-3tcos6t+96sin6tut

Look up the inverse transform of the individual terms after PFE:

- Kr⋅δntLKr⋅sr

- Rk⋅epktutLRks-pk

- S2⋅tepktutLS2s-pk2

Example: Hs=5ss+2s+12, Res>-1

Hs=c1s+2+c2s+1+c3s+12=5ss+2s+12

c1=5ss+12s=-2=-10

c3=5ss+2s=-1=-5

-10s+2-5s+12+c2s+1=-10s+12-5s+2+c2s+2s+1s+2s+12

-10s2+2s+1-5s-10+c2s2+3s+2=-10+c2s2+-20-5+3c2s+-10-10+2c2=5s

c2=10

Hs=-10s+2-5s+12+10s+1

ht=-10e-2t+10e-t-5te-tut

Example: Hs=s2+3s+1s+1s2+2s+2, Res>-1

s2+3s+1s+1s2+2s+2=s2+3s+1s+1s+12+1=c1s+1+c2s+1+c3s+12+1

c1=s+1Hss=-1=s2+3s+1s2+2s+2s=-1=-1

c1s+12+c1+c2s+12+c3s+1=c1+c2s2+2s+1+c1+c3s+c3=c2-1s2+2c2+c3-2s+c2+c3-2=s2+3s+1

c2=2

c3=1

Hs=-1s+1+2s+1+1s+12+1

ht=-e-tut+2e-tcostut+e-tsintut

Because the poles are either real or complex conjugates, the impulse response is real.

### Analyzing LCC differential equations

k=0Nakdkytdtk=k=0Mbkdkxtdtk

Assuming zero initial conditions, take the LT of both sides to get the system function Hs. Differentiation theorem applies here.

k=0NakskYs=k=0MbkskXs

Hs=YsXs=k=0Mbkskk=0Naksk

Analysis:

- Plot the poles and zeros of Hs. If the poles are in the LHP, the system is stable.

- If the system is stable, plot the frequency response Hss=j2πf=jw

- Find the inverse LT L-1Hs to get the impulse response ht

- Find L-1LutHs=L-1Hss to get the step response

- Find L-1XsHs to get the output yt in response to xt

Nonzero initial conditions:

A feature of the one-sided LT is that you can handle nonzero initial conditions when solving the general LCC differential equation.

Nonzero initial conditions are carried by the terms xk0-=dkxtdtkt=0-.

Example: let the LTI system input be xt=2e-5tut and the system is characterized by y't+3yt=xt, y0-=2.

You still apply differentiation theorem to both sides of the LCC differential equation.

sYs-y0-+3Ys=Xs

s+3Ys=Xs+y0-

Ys=Xss+3+2s+3=2s+5s+3+2s+3=R1s+5+R2s+3+2s+3

R1=2s+3+2s+5s+3s=-5=-1

R2=2s+5s=-3=1

Ys=-1s+5+1s+3+2s+3→yt=-e-5tut+3e-3tut

The first term is the forced response (aka particular solution) because it has the same exponential term as the input xt.

The second term is the transient or natural response (aka homogeneous solution) because it has the term e-3t, which is also the natural mode of the homogeneous equation y't+3yt=0. This response arises from both the input and from the initial condition.

In this case, both components decay to 0 since the input itself is decaying.

Frequency response from poles and zeros:

Hjw=k=0Mbkskk=0Naksks=jw=k=1Ms-zkk=1Ns-pks=jw=k=1MVzkjwk=1NVpkjw

Vzk=jw-zk and Vpk=jw-pk are the frequency response contributions of each zero and pole. The vectors point from the zero/pole locations to the jw axis.

Hjw=k=1MVzkjwk=1NVpkjw

∠Hjw=k=1M∠Vzkjw-k=1N∠Vpkjw

# z-transform

- Generalization of DTFT

- Applies to a broader class of signals than DTFT

- More user-friendly than DTFT because LCC difference equations can be solved by just algebraic manipulation

- Function of a complex variable, z

- Closely parallels Laplace transform

## Two-sided (bilateral) z-transform

Xz=Zxn=n=-∞∞xnz-n

Xz is the ZT of xn. When xn is the impulse response of an LTI system, Xz is also called the system function.

Relationship to DTFT:

Xzz=rejw=nxnr-ne-jwn=Fxnr-n

The DTFT is equal to the ZT evaluated on the unit circle in the complex z plane, r=1.

Xejw=Xzz=ejw

### ROC

ZT converges when xnr-n is absolutely summable.

Xz≤nxnz-n=nxnz-n=nxnr-n<∞

Convergence depends only on r, so if the series converges for z1, then the series converges for the circle defined by z=z1.

The ROC of the ZT is an annulus (a ring). If it contains the unit circle, then Xejw exists.

Right-sided sequence:

A right-sided sequence extends into +∞ and is 0 for n<no, where no can be positive or negative.

xan=anun

Xaz=nxanz-n=n=0∞anz-n=n=0∞az-1n=11-az-1=zz-a

az-1<1→z>a

Left-sided sequence:

A left-sided sequence extends into -∞ and is 0 for n>no, where no may be positive or negative.

xbn=-anu-n-1

Xbz=nxbnz-n=-n=-∞-1anz-n=-n=-∞-1az-1n=-l=1∞az-1-l=-l=1∞a-1zl=--1+l=0∞a-1zl=1-11-a-1z=a-1za-1z-1=zz-a=11-az-1

a-1z<1→z<a

xa and xb have the same ZT, except for the ROCs, which are complements.

### Poles, zeros, and stability

LTI system with hn and system function Hz.

These are equivalent conditions for the LTI system being BIBO stable:

- hn is L1 or absolutely summable

- Hejw exists and converges pointwise

- ZT ROC contains unit circle, z=ejw

- The system is BIBO stable

Remember, the ROC is defined by r s.t.

nhnr-n<∞

If it includes the unit circle, then

nhn<∞

The poles of Hz are outside of the ROC because Hz→∞ at the poles. In the ROC,

Hz≤nhnr-n<∞

For right-sided sequences, the ROC is the exterior of a circle with radius corresponding to the largest pole radius. Therefore, if your system is causal, its poles must lie within the unit circle for the system to be stable.

Example: hn=13nun+12nun

Hz=11-13z-1+11-12z-1=2-56z-11-13z-11-12z-1=2z2-56zz-13z-12=2zz-512z-13z-12, z>12

### Finite-length sequences

If hn is finite length and has finite values, then Hz converges for all z except possibly 0 and ∞.

If Hz contains negative powers of z, then the ROC excludes z=0. If it contains positive powers of z, then the ROC excludes z→∞.

If hn is nonzero only for N1≤n≤N2, then

Hz=xN1z-N1+⋯+xN2z-N2

If hn is causal, then N1,N2≥0 and Hz contains only nonnegative powers of z.

Example: xn=anun-un-N

Xz=n=0N-1anz-n=n=0N-1az-1n=1-az-1N1-az-1=zN-aNzN-1z-a

Numerator:

zN=aN=aNej2πk→z=aej2πkN, k∈0,N-1

When k=0, the pole and zero at a cancel out. Therefore,

- Zeros: z=aej2πkN, k∈1,N-1

- Poles: z=0 (multiplicity N-1)

Since xn is causal, ROC is z>0.

When the ZT is a ratio of polynomials in z or z-1, the number of poles is equal to the number of zeros. Poles and zeros may be at infinity.

For example, consider Hz=z-a. There is a zero at z=a, but there is also a pole at z→∞ since limz→∞z-a=∞.

Or Hz=1z-a. There is a pole at z=a, but there is also a zero at z→∞ since limz→∞1z-a=0.

Example: xn=δn-32δn-1-δn-2

Xz=1-32z-1-z-2=z2-32z-1z2

zeros=32±94+42=34±916+1616=34±54=2, -12

poles=0, 0

Xz=z-2z+12z2, z>0

Now shift by +1: x1n=xn+1=δn+1-32δn-δn-1

X1z=zXz=z-2z+12z, 0<z<∞

There is one pole at 0 and one pole at ∞.

### Inverse ZT

Like LT, the formal inverse ZT requires contour integration over z (integrate on a closed curve in the z-plane instead of using an ordinary integral along a line).

Most practical way is to use PFE and table lookup (like LT).

We assume that Xz takes this form:

Xz=NzDz=k=0Mbkz-kk=0Nakz-k

For PFE, Xz must be a proper rational, N>M. If it’s not proper, use long division to put it in this form:

Xz=r=0M-NKrz-r+N1zDz

Where N1zDz is proper.

Distinct poles (no repeats):

When Xz contains only simple poles, it takes the form

Xz=k=1NRk1-pkz-1

Rk are the PFE coefficients and pk are the nonzero poles. Use the residue formula to find the coefficients.

Twin poles:

When pi is repeated once (multiplicity of two), then

Xz=S11-piz-1+S21-piz-12+k=1,k≠iNRk1-pkz-1

Use residue formula to find Rk, then find S2, then find S1.

Performing inverse LT:

After PFE, use table lookup. If all signals are causal, we have

- Krδn-rZKrz-r, z>0 for r>0

- RkpknunZRk1-pkz-1, z>pk

- S2n+1pknunZS21-pkz-12, z>pk

Example: Xz=1+2z-1+z-21-12z-11+14z-1, z>12

1+2z-1+z-21-12z-11+14z-1=1+2z-1+z-21-14z-1-18z-2=-8+91-12z-11+14z-1

91-12z-11+14z-1=A1-12z-1+B1+14z-1

A=91+14z-1z=12=91+12=6

B=91-12z-1z=-14=91+2=3

Xz=-8+61-12z-1+31+14z-1, z>12

xn=-8δn+612nun+3-14nun

Example: Xz=11-12z-11-14z-12, z>12

Xz=A1-12z-1+S11-14z-1+S21-14z-12

A=11-14z-12z=12=11-122=4

S2=11-12z-1z=14=11-2=-1

41-12z-1+S11-14z-1-11-14z-12=11-12z-11-14z-12

Numerator:

41-14z-12+S11-12z-11-14z-1-1+12z-1=41-12z-1+116z-2+S11-34z-1+18z-2-1+12z-1=1

4+S1-1=1→S1=-2

xn=412nun-214nun-n+114nun

### Theorems and properties

xnZXz, ROC=Rx

When combining signals, ROC may be larger than the intersection of the individual ROCs due to pole-zero cancellation.

| Property | xn | Xz | ROC |
| --- | --- | --- | --- |
| Linearity | ax1n+bx2n | aX1z+bX2z | At least Rx1Rx2 |
| Delay (time shift) | xn-n0 | z-n0Xz | Rx, maybe excluding 0 or ∞ |
| Differentiation | nxn | -z⋅dXzdz | Rx, maybe excluding 0 or ∞ |
| Conjugation | x*n | X*z* | Rx |
| Convolution | x1n*x2n | X1zX2z | At least Rx1Rx2 |
| First difference | xn-xn-1 | 1-z-1Xz | At least Rxz>0 |
| Accumulation | k=-∞nxk | 11-z-1Xz | At least Rx⋂z<∞ |
| Initial value | x0=limz→∞Xz xn is causal |  |  |

### Transform pairs

| Name | xn | Xz | ROC |
| --- | --- | --- | --- |
| Impulse | δn | 1 | All z |
| Delayed impulse | δn-n0 | z-no | All z except 0 or ∞ |
| Decaying exponential | anun | 11-az-1 | z>a |
| Decaying exponential, left-sided | -anu-n-1 | 11-az-1 | z<a |
| Twinned pole | n+1anun | 11-az-12 | z>a |
| Damped cosine | rncoswonun | 1-rcoswoz-11-2rcoswoz-1+r2z-2 | z>r |
| Damped sine | rnsinwonun | rsinwoz-11-2rcoswoz-1+r2z-2 | z>r |
| Windowed decaying exponential | anun-un-N | 1-aNz-N1-az-1=zN-aNzN-1z-a | z>0 |

### Analyzing LCC difference equations

The general LCC difference equation describes a causal LTI system.

k=0Nakyn-k=k=0Mbkxn-k

Take ZT of both sides to get the system function.

k=0Nakz-kYz=k=0Mbkz-kXz

Hz=YzXz=k=0Mbkz-kk=0Nakz-k

With the system function, you can

- Get the impulse response: Z-1Hz=hn

- Get the step response: Z-1ZunHz=Z-1Hz1-z-1=sn

- Get the output response to xn: Z-1XzHz=yn

Example: xn=412nun and Hz=1+13z-11-14z-1, z>14

Xz=41-12z-1,z>12

Yz=41+13z-11-12z-11-14z-1, z>12

41+13z-11-12z-11-14z-1=A1-12z-1+B1-14z-1

A=41+13z-11-14z-1z=12=45312=403

B=41+13z-11-12z-1z=14=-473=-283

yn=40312nun-28314nun

Frequency response from poles and zeros:

Hejw=Hzz=ejw=k=0Mbke-jwkk=0Nake-jwk=G⋅zNzM⋅k=1Mz-zkk=1Nz-zpz=ejw=k=1MVzkejwk=1NVzpejw

Vzk=ejw-zk

Vpk=ejw-zp

Vzk and Vzp are the contributions of each zero and pole to the overall frequency response. They are vectors that point from a pole/zero to the unit circle.

The ratio of the vector magnitudes gives the magnitude response. The sum/difference of the vector phases gives the phase response.

Hejw=k=1MVzkejwk=1NVpkejw

∠Hejw=k=1M∠Vzkejw-k=1N∠Vpkejw

# Analysis and modeling across domains

System architecture will likely contain both CT and DT subsystems. LCC differential and difference equations are building blocks for these subsystems; they are responsible for implementing filters.

For design and analysis, you can view filters in the time, frequency, s, and z domains.

Problem-solving steps:

- Create a system block diagram from the problem statement

- Create detailed subsystem models for critical areas in the design

- Start with simple behavioral level models for perceived non-critical subsystems

- Through analysis, get a first-cut design of the critical subsystems

- Verify composite system performance via a high-level analysis model and simulation

## Continuous time

Three representations of LCC differential equations: time, frequency, and s. They all use the same coefficient sets, bk and ak.

Time domain, differential equation:

k=0Nakykt=k=0Mbkxkt

Time domain, impulse response:

yt=-∞∞xτht-τdτ

Frequency domain:

Yf=XfHf

Hf=k=0Mbkj2πfkk=0Nakj2πfk

HjΩ=k=0MbkjΩkk=0NakjΩk

s domain:

Ys=XsHs

Hs=k=0Mbkskk=0Naksk

## Discrete time

Three representations of LCC difference equations: time, frequency, and z. They all use the same coefficient sets, bk and ak.

Time domain, difference equation:

k=0Nakyn-k=k=0Mbkxn-k

Time domain, impulse response:

yn=k=-∞∞xkhn-k

Frequency domain:

Yejw=XejwHejw

Hejw=k=0Make-jwkk=0Nbke-jwk

z domain:

Yz=XzHz

Hz=k=0Makz-kk=0Nbkz-k

## Analog filter design example

Design an analog Butterworth lowpass filter meeting the following specifications:

- Passband frequency: fp=1kHz

- Maximum passband attenuation: Ap=3dB

- Stopband frequency: fs=10kHz

- Minimum stopband attenuation: As=50dB

Find the step response, the peak overshoot, and time of the peak overshoot.

Butterworth filter has the form

Hf=11+ffc2N

Hs=1k=1Ns-pk

Where N is the filter order and fc is the 3dB cutoff frequency.